Electrical Tests of SCT Hybrids and Modules

Version 3.11

21.01.02

Peter W. Phillips Rutherford Appleton Laboratory

Lars Eklund Uppsala University

Abstract

This document aims to describe each of the electrical (readout) tests of SCT hybrids and modules that will be performed during production. A number of test sequences designed to simplify the task at hand are also presented.

The methodology of each test and the subsequent analysis of its data are outlined.  A number of possible defect types that can be identified by each test is given, and a table of values to be recorded in the ATLAS SCT production database is defined.  In each case an example is included of the ASCII file output by the analysis software and designed to be read into the production database.

TABLE OF CONTENTS

System Overview	3

Introduction	3

Test results and the SCT Database	4

The Format of the Results File	5

Sequences and Optional Extras	7

Before Running a Sequence	7

CharacterisationTest.cpp	7

ConfirmationTest.cpp	8

Other Electrical Tests	8

Description of DC Tests	9

IVCurve.cpp	9

Description of Digital Tests	11

HardReset.cpp	11

RedundancyTest.cpp	13

FullBypassTest.cpp	16

PipelineTest.cpp	18

Description of Analogue Tests	21

StrobeDelay.cpp	21

ThreePointGain.cpp	24

TrimRange.cpp	28

ResponseCurve.cpp	34

NO.cpp	38

Timewalk.cpp	41

Description of Other Electrical Tests	44

HybridLTT.cpp	44

ModuleLTT.cpp	46

Appendices	47

Appendix 1 – Definitions of Utility Tables	47

Appendix 2 – Definitions of Result Tables	48

Appendix 3 – List of Defect Names	53

Appendix 4 – Database table implementation status	54

References	54

# System Overview

## Introduction

This section outlines the mechanism used to run each test, right through to the point at which the data has been recorded in the database.  Use of MuSTARD, SLOG and SCTLV hardware has been assumed, together with the SCTDAQ software package.  Ideally a CLOAC module would be used, particularly for studies of noise occupancy, but in all cases a sensible backup option has been provided using only SLOG.  SCTHV may also be used to provide detector bias to the modules under test.

JAVA

application

Database

Results File

(Text)

Analysis Macro

Test Macro

ST.cpp

ROOT

CINT

stdll

Sequence Macro

RAW DATA

(ROOT FILE)

Plots

(Postscript)

Histograms

(ROOT FILE)

CLOAC

MuSTARD

SLOG

SCTHV

SCTLV

VME

The basic routines to communicate with each VME board are implemented in the form of a static library written in C.   Higher level functions have been implemented in a small number of C++ classes that are linked together with the static libraries, and some of the ROOT libraries, to form the dynamic link library stdll.  This is the full extent of the compiled code: everything else runs through the CINT interpreter.

Within ROOT, the user runs the top-level macro ST.cpp to load stdll, to initialise the system and to present the system menu.  Each of the proposed electrical tests has been implemented in the form of a discrete ROOT macro which may be run by pressing a button within the menu system, by running an overall “sequence macro” or by direct user input at the command prompt.  At the end of each scan a ROOT file is generated which contains the scan data for each module, records of error and event counters and some module configuration and DCS information.

Analysis of the raw data from each test is performed by a separate root macro which appends results to a text file, one per hybrid or module per day, such that the results of each test in the sequence are kept together in one file.  It is foreseen that one or more Java applications will be written to upload this data and any associated raw data files directly into the database.  For most tests, results may also be output in the form of plots within a postscript file.  In future the software may be revised to keep a number of the derived histograms for each module together in a separate root file.

## Test results and the SCT Database

The following table is reproduced from the list of SCT Database table definitions.

TESTS:	General Details of Tests on Items

| Column Name | Type | Description |
| --- | --- | --- |
| SER_NO | Long int | Atlas serial no. of item |
| TEST_NO | Long int | ID no. of test |
| RUN_NO | Char(80) | Data taking run no. |
| TEST_NAME | Char(20) | Name of test |
| TEST_DATE | Date | Date of test |
| LOCN_NAME | Char(50) | Location of test |
| INITLS | Char(4) | Initials of user |
| PASS | Boolean | Passed unconditionally? |
| PROBLEM | Boolean | Problems encountered? |
| SUBSYST | Char(1) | Subsystem code |
| LAST_MOD | Date | Date record last updated |
| OWNER | Char(30) | Owner institute of the record |
| Primary Key: | TEST_NO |  |
| Foreign Key: | SER_NO | ITEMS(SER_NO) |
| Foreign Key: | TEST_NAME | TEST_DESCR(TEST_NAME) |
| Foreign Key: | LOCN_NAME | LOCNS(LOCN_NAME) |
| Foreign Key: | INITLS | PERSONS(INITLS) |

For each electrical test performed upon a hybrid or module an instance of the above object will be added to the database, hence the serial number, run number, location and tester are always recorded.

Digital tests will set the “PASS” field to “NO” if more than a specified number of defects are found, but in general the analogue tests do not make use of this field. The “PROBLEM” field is not used by the electrical test software and is set to “NO” in all cases.  The remaining fields shown in the above table are for internal use by the database.

Information specific to each electrical test is recorded in one or more data tables, defect objects and/or raw data files, all of which are liked to the relevant instance of the basic test object described above.

A number of utility tables have been designed to store information that it is desired to record for a number of tests, such as monitored DCS parameters.  This has greatly simplified the definition of the tables needed to store the results of each specific test, since all common factors have been removed. The definitions of the utility tables can be found in appendix 1.

In addition to the utility tables, a table has been designed to record the results of each test.  These tables are defined in appendix 2.  There is a direct link to each table from the relevant section of the text.

The list of defects that may be identified by each test are detailed in the test and summarised in appendix 3.

Additional configuration information or more detailed results may be stored in the form of (ASCII) raw data.

A list of all database tables specific to the electrical testing of hybrids and modules, showing their implementation status, can be found in appendix 4.

## The Format of the Results File

Results from each test are appended to a file:

sctvar\results\serialnumber_YYYYMMDD.txt

For example:

sctvar\results\20220170100016_20010821.txt

The following tag is used to indicate the start of a new section, corresponding to a new test:

#

%NewTest

This is followed by a section containing information stored in the basic test entity:

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-11

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

Next comes a section describing the IP name of the host computer, the software version number and the time of the test, corresponding to the utility table TSTDAQINFO:

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.11”

#DUT

“Barrel Module”

#TIME

“18:15:18”

The next section gives the monitored DCS values, corresponding to the utility table TSTDCSINFO:

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	Icc

3.500	790

#VDD	IDD

- 535

#TIME_POWERED

.

Depending upon the test which has been executed, there may now follow a section giving details of the number of scans used to make a measurement and the corresponding fixed parameter used for each of those scans.  This information is stored in the utility table TSTSCANINFO.  The following example lists the number of scans and charges used to make a three point gain measurement:

#

%SCAN_INFO

#

#POINT TYPE

“QCAL (fC)”

#N_POINTS

3

#POINTS

1.50	2.00	2.50

The actual results of the test are summarised in the following section, corresponding to the table specific to that test.  By way of an example, here is the format used to list the results of the StrobeDelay test:

#

%StrobeDelay

#

#DELAY

#M0 S1  S2  S3  S4  E5

-1  –1  –1  –1  16  17

#

#M8 S9  S10 S11 S12 E13

17  15  16  16  16  18

#

A list of defects is generated if any defects were found by the test.  The types of defect that are identified by each test are described in the corresponding section of this document and summarised in appendix 3.  Each defect is prefaced by the tag “%Defect”.

Here is a list of defects found by the StrobeDelay test:

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 0

LAST CHANNEL	: 127

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 128

LAST CHANNEL	: 255

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 256

LAST CHANNEL	: 383

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 384

LAST CHANNEL	: 511

#

#4 defects found

#

Finally, any raw data files which are to be associated with the test are listed.

%TEST Rawdata

FILENAME	: D:\sctvar\results\20220170100016_rc_246-12.txt

Further examples are given throughout the remainder of the document.

# Sequences and Optional Extras

## Before Running a Sequence

The following tests do not apply to all devices, hence they have been omitted from the characterisation and confirmation sequences.  It is left to the user to run them manually as required by means of the menu system.

### DC Test:

- IVCurve.cpp (modules only)

### Digital Tests:

- StreamDelay.cpp (implemented but not yet documented)

- Opto Functionality Tests (forward only) (not yet implemented)

## CharacterisationTest.cpp

The Characterisation Sequence aims to verify the basic digital functionality of a hybrid or module, and to fully characterise its analogue performance.  The menu system contains a button to start the sequence, which comprises the following tests:

### Digital Tests:

- HardReset.cpp

- RedundancyTest.cpp

- FullBypassTest.cpp

- PipelineTest.cpp

### Analogue Tests:

- StrobeDelay.cpp

- ThreePointGain.cpp (untrimmed)

- TrimRange.cpp

- ResponseCurve.cpp

- NO.cpp

- Timewalk.cpp

## ConfirmationTest.cpp

The Confirmation Sequence aims to verify, as quickly as possible, that the performance of a hybrid or module has not deteriorated, perhaps as a result of handling or shipping.

The digital tests that are part of this sequence are chosen to help identify damaged wirebonds and the analogue tests, whilst sufficient to demonstrate that the basic performance is as expected, avoid the more time consuming tasks such as trimming or a measurement of the noise occupancy.  The menu system contains a button to start the sequence, which comprises the following tests:

### Digital Tests:

- HardReset.cpp

- RedundancyTest.cpp

- FullBypassTest.cpp

### Analogue Tests:

- StrobeDelay.cpp

- ThreePointGain.cpp (trimmed or untrimmed, as appropriate)

## Other Electrical Tests

Some of the tests listed in this section are mandatory, such as the long term test of the hybrid, and some are optional, such as a laser scan of the finished module.  Not all of these tests have yet been written: in any case they are shown here as placeholders.

- HybridLTT.cpp (provisional version available)

- ModuleLTT.cpp (provisional version available)

- Laser Efficiency Scan (not yet implemented)

- Source Efficiency Scan (not yet implemented)

# Description of DC Tests

## IVCurve.cpp

This test has been designed for users of the SCTHV high voltage power supply module.  Those without access to this module may wish to perform a similar test using other hardware.  If the results are stored in the same format, they may still be uploaded to the database.

### Method

An IV curve is taken raising the detector bias voltage from 10V to an upper limit of 150V, 350V or 500V as selected by the operator, always in 10V steps.  (It is recommended that the number of times for which the module bias is taken to 500V should be kept as low as is possible).

A firmware current limit of 100 A, set high to allow for charging currents, is imposed throughout the sequence. If this limit is exceeded, the voltage will trip off.  The current drawn at each voltage point is recorded 10 seconds after the requested voltage has been reached.  In addition, should a module draw more than 10 A current after the 10 second wait time, the voltage applied to that module will not be raised further.  Upon completion of the test, the bias voltage is turned down to 150V, the standard used during analogue tests of unirradiated modules.

During production testing this test would usually be performed before the chip bias has been applied but there may be exceptions to this rule, hence it is necessary to record the voltages supplied to, and the currents drawn by, the hybrid.  This is achieved by sampling all monitored DCS parameters when the bias voltage has reached its highest point.  The information is recorded within an instance of the database table TSTDCSINFO.

### Analysis

If a current drawn by a module exceeds the lower limit of 10A, the defect IV_LIMIT is recorded.  If the high voltage supply connected to a module trips off, the defect IV_TRIP is recorded.

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

##### TSTDCSINFO:		monitored voltages, currents and temperatures

##### TSTMODIV:		leakage currents at 150V and 350V

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| IV_LIMIT | 0 | 1535 |
| IV_TRIP | 0 | 1535 |

RAW DATA:

ABCD_CONFIG (optional)

IV_DATA (mandatory)

### Acceptance

The database reporting structure should also provide the possibility to set stricter acceptance cuts on the range of acceptable detector bias current values, and to compare values against the sum of the currents drawn by the four detectors before assembly into a module.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“16:20:02”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

0.000	0

#VDD	IDD

- 0

#TIME_POWERED

.

#

%ModIVScan

#

TEMPERATURE	: 25.5

I LEAK 150	: 0.994

I LEAK 350	: 1.220

#

#No defects found!

#

%TEST Rawdata

FILENAME	: D:\sctvar\results\20220170100016_iv_20010824.txt

# Description of Digital Tests

## HardReset.cpp

### Method

- The power is cycled such that all chips return to the power on condition.  With the aid of an oscilloscope and following the guidance given by the software, the user must verify that each Master chip outputs clock divided by 2.  Once this has been completed the currents Icc and Idd are recorded (ICC_NOCONFIG, IDD_NOCONFIG).

- A series of configuration commands is issued to set the clock feedthrough bit high and to program all DACs to nominal values as shown in the table below.  The user is prompted to confirm that output of clock/2 by each Master chip has stopped. The currents Icc and Idd  drawn in this configuration are recorded within an instance of the TSTDCSINFO table.

- The HARD RESET signal is issued. The user is prompted to check that once more each master chip outputs clock/2.

- The clock is momentarily turned off, and the analogue and digital currents Icc and Idd are recorded (ICC_NOCLOCK, IDD_NOCLOCK).

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | ON |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | 01X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | None |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | 100mV |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | 10mV (1.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | - |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | 0 |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

If either datalink should fail to output the clock/2 signal upon power up, that will be recorded as the defect HR_NOCLK.  If clock/2 output does not cease upon transmission of the configuration sequence, the defect HR_NOCON is recorded.  Finally, if clock/2 does not return after the HARD RESET signal, that is recorded as the defect HR_NORST.

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

##### TSTDCSINFO:		monitored voltages, currents and temperatures

TSTHYBRESET:	digital and analogue currents when not configured or when clock interrupted

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| HR_NOCLK | Link * 768 | ((Link+1) *768) -1 |
| HR_NOCON | Link * 768 | ((Link+1) *768) -1 |
| HR_NORST | Link * 768 | ((Link+1) *768) -1 |

RAW DATA:

ABCD_CONFIG (optional)

### Acceptance

None of the defects found by this test are allowable on production modules, hence the “PASS” field will be set to “NO” if any defects are found.

The database reporting structure should also provide the possibility to set acceptance cuts on the range of allowable current values Icc and Idd in each of the three cases.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“16:28:26”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%HardReset

#NOCONFIG	ICC	IDD

- 525

#NOCLOCK	ICC	IDD

795	260

#

#No defects found!

#

#No raw data!

## RedundancyTest.cpp

The hybrid or module is configured as described below by use of each clock/command pair.  Since this test also aims to check the integrity of command reception, one of a number of defined mask patterns is written to each chip prior to each L1A trigger.  Consecutive events use different mask patterns.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | OFF |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | X1X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | MASK + Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | (variable) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | (variable) - SEE BELOW |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | 500mV |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | 10mV (1.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | - |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Full Configuration + Soft Reset + 129 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### LOOPS | 1 – SELECT | 0,1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 – MASK | Mask #1, #2 |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | 100 |  |  |  |  |  |  |  |  |  |  |  |  |

The mask patterns to be used are as follows:

- a sequence of 010101…01

- a sequence of 101010…10

Soft Reset is issued a specified number of BCOs before each L1A to ensure that each event is shifted through the same cells of the pipeline.  In this way any channel affected by a dead cell in the pipeline can only have zero occupancy, and any difference in the occupancy of a channel for each setting of SELECT must have been caused by addressing or command reception errors.

### Analysis

Each channel is categorised according to occupancy as follows:

| nhits 0 (select = 0) | nhits 1 (select = 1) |  |
| --- | --- | --- |
| 50 | 50 | Good channel |
| 100 | 100 | Stuck channel |
| 0 | 0 | Dead channel (not yet able to resolve if pipeline or mask defect) |
| 50 | 0 or 100 | ID4 addressing error when select = 1 |
| 0 or 100 | 50 | ID4 addressing error when select = 0 |
| 50 | 0 < n < 100 | Command reception is not reliable when select = 1 |
| 0 < n < 100 | 50 | Command reception is not reliable when select = 0 |
| other | other | Command reception is never reliable |

During the second stage of the analysis the numbers of channels of each chip which fall into each of the above categories is tallied leading to the categorisation of each chip as shown below.  All failures are recorded as defects.

IF ((nch_good>120) AND (remainder STUCK OR DEAD))

PASS code 0

ELSE IF (nch_addressing_error_0>120)

FAIL code 1 (defect CLK_ADDR0)

ELSE IF (nch_addressing_error_1>120)

FAIL code 2 (defect CLK_ADDR1)

ELSE IF (nch_command_error_0>120)

FAIL code 3 (defect CLK_COM0)

ELSE IF (nch_command_error_1>120)

FAIL code 4 (defect CLK_COM1)

ELSE	FAIL code 5 (defect CLK_ERROR)

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

##### TSTDCSINFO:		monitored voltages, currents and temperatures

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| CLK_ADDR0 | Chip * 128 | ((Chip+1) * 128) -1 |
| CLK_ADDR1 | Chip * 128 | ((Chip+1) * 128) -1 |
| CLK_COM0 | Chip * 128 | ((Chip+1) * 128) -1 |
| CLK_COM1 | Chip * 128 | ((Chip+1) * 128) -1 |
| CLK_ERROR | Chip * 128 | ((Chip+1) * 128) -1 |

RAW DATA:

ABCD_CONFIG (optional)

### Acceptance

None of the defects found by this test are allowable on production modules, hence the “PASS” field will be set to “NO” if any defects are found.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-7

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:15:18”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%RedundancyTest

#

#RedundancyTest Summary - not for the database

#chip pass good stuck dead adr0 adr1 com0 com1 com

# 0  1  128    0    0    0    0    0    0    0

# 1  1  128    0    0    0    0    0    0    0

# 2  1  128    0    0    0    0    0    0    0

# 3  1  128    0    0    0    0    0    0    0

# 4  1  128    0    0    0    0    0    0    0

# 5  1  128    0    0    0    0    0    0    0

# 6  1  128    0    0    0    0    0    0    0

# 7  1  128    0    0    0    0    0    0    0

# 8  1  128    0    0    0    0    0    0    0

# 9  1  128    0    0    0    0    0    0    0

#10  1  128    0    0    0    0    0    0    0

#11  1  128    0    0    0    0    0    0    0

#

#No defects found!

#

#No raw data!

## FullBypassTest.cpp

The module is programmed to each possible configuration with regard to the routing of token and data between the chips. Since the irradiation programme has shown that different chips can have different susceptibilities to radiation damage effects with regard to the token passing, this test is performed over a range of digital voltages, Vdd.  In this way the minimum value of Vdd required for the correct operation of each bypass link can be determined, and any inherent difference in the performance of the various links is known upon production.

The number of configurations to be tested is greater for forward hybrids than for barrel hybrids since more redundancy links have been implemented in the forward design.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | (variable) SEE BELOW |
| --- | --- | --- | --- |
|  |  | Edge Detect | OFF |
|  |  | Data Compression | X1X |
|  |  | Mode | MASK + Data Taking Mode |
|  |  | Select | 0 |
|  |  | Masked Channels | None |
|  | ##### DACs | Ipreamp | 220A |
|  |  | Ishaper | 30A |
|  |  | Threshold DAC | 500mV |
|  |  | Calibration DAC | 10mV (1.0fC) |
|  |  | Delay Register | - |
|  |  | TrimRange | 0 |
|  |  | TrimDAC | 0 |
|  | ##### TRIGGER | Sequence | L1A |
|  |  | Frequency | - |
| SCANS | ##### LOOPS | 1 – Vdd | 4.0, 3.9, 3.8, 3.7, 3.6, 3.5 |
|  |  | 2 – Configuration | Barrel hybrid or module: cases 0 – 35 Forward hybrid or module: cases 0 – 62 |
|  | n Triggers | 100 |  |

### Analysis

For each configuration, the number of dead channels of each chip is tallied.  Chips having more than 120 dead channels are considered to have returned no data, otherwise they are considered to have returned good data.  This matrix is then compared with expectation to determine the minimum value of Vdd needed for the correct operation of each bypass link.

Any bypass links that do not function correctly when Vdd is set to the minimum value of 3.5V are recorded as DEFECTS.  In this case Vdd_min is recorded as 10.0V.  In the case where a link has not been tested, Vdd_min is recorded as 0V.  A comment is recorded in each case.

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

##### TSTDCSINFO:		monitored voltages, currents and temperatures

TSTHYBBPASS:	minimum Vdd required for operation of each token/bypass link

DEFECTS:

| Defect | First Chan | Last Chan |
| --- | --- | --- |
| TOKEN | Chip * 128 | ((Chip+1) * 128) -1 |
| RTOKEN | Chip * 128 | ((Chip+1) * 128) -1 |

RAW DATA:

ABCD_CONFIG (optional)

### Acceptance

A global “PASS” will be recorded if no defects are found.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-7

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:15:06”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%FullBypassTest

#

#Vmin

#token	r_tkn	token	r_tkn

#M0 		S1

  - 3.5	3.5	3.5

#S2		S3

3.5	3.5	3.5	3.5

#S4		E5

3.5	0.0	0.0	3.5

#M8		S9

3.5	3.5	3.5	3.5

#S10		S11

3.5	3.5	3.5	3.5

#S12		#E13

3.5	0.0	0.0	3.5

#

#Comment

#token	r_tkn	token	r_tkn

#M0		S1

“Minimal tested”	“Minimal tested”	“Minimal tested”	“Minimal tested”

#S2		#S3

“Minimal tested”	“Minimal tested”	“Minimal tested”	“Minimal tested”

#S4		#E5

“Minimal tested”	“Not tested”	“Not tested”	“Minimal tested”

#M8		#S9

“Minimal tested”	“Minimal tested”	“Minimal tested”	“Minimal tested”

#S10		S11

“Minimal tested”	“Minimal tested”	“Minimal tested”	“Minimal tested”

#S12		E13

“Minimal tested”	“Not tested”	“Not tested”	“Minimal tested”

#

%DEFECTS

#name	bypass_link	Vdd_min

#No defects found!

## PipelineTest.cpp

The cells of the ABCD3T pipeline are tested in order to identify any cells which may permanently output zero (dead) or one (stuck).  (Although the pipeline has already been tested during wafer probing, it may be desired to repeat this test to identify any new failures.) The pulse input register facility is also tested.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | OFF |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | X1X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | None |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | 500mV |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | 10mV (1.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | - |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Soft Reset + n BCO delay + Pulse Input Register Command + 129 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – MASK | Mask #0, #3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 – n BCO delay | 150 to 161, step size = 1 |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | 100 |  |  |  |  |  |  |  |  |  |  |  |  |

The mask patterns to be used are as follows:

- no channels masked (to identify dead cells)

- all channels masked (to identify stuck cells)

### Analysis

The number of channels that return data for each scanpoint is tallied – corresponding to the number of good channels.  If for a given channel all 12 cells are found to be dead or stuck, then a defect type of DEAD or STUCK is recorded.  If for any channel the number of dead cells or stuck cells is greater than zero but less than 12, each individual defect is noted as either type DEADCELL or STUCKCELL.

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTHYBPIPE:		the number of good pipeline cells in each chip

DEFECTS:

| Defect | First Chan | Last Chan |
| --- | --- | --- |
| DEAD | CHANNEL | CHANNEL |
| STUCK | CHANNEL | CHANNEL |
| DEADCELL | CHANNEL | CHANNEL |
| STUCKCELL | CHANNEL | CHANNEL |

RAW DATA:

ABCD_CONFIG (optional)

### Acceptance

None of the defects found by this test are allowable on production modules, hence the “PASS” field will be set to “NO” if any defects were found.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-8

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:15:25”

#

%DCS_INFO

#

#T0	T2

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%PipelineTest

#

#NGOOD

#M0 S1  S2  S3  S4  E5

128 128 128 128 128 128

#

#M8 S9  S10 S11 S12 E13

128 128 128 128 128 128

#

#No defects found!

#

#No raw data!

# Description of Analogue Tests

## StrobeDelay.cpp

This test aims to establish the optimum setting of the delay register for each chip, to be used during all subsequent analogue tests.  The size of each delay step is known to vary as a function of temperature, hence the strobe delay setting must always be re-optimised each time a device is operated at a different temperature.  Sufficient time must always be allowed for the powered, configured hybrid or module to reached thermal equilibrium with its surroundings before this test may be undertaken.

Running with Edge Detect ON, compression 01X, a 4.0fC signal is injected for a threshold of 2.0fC.  A functional fit is made to both the rising and falling edges to give the points at which 50% efficiency is achieved.  The operating point is chosen to be 25% of the distance between these two points with reference to the rising edge of the strobe delay peak, which corresponds to the falling edge of the signal. By setting the timing some distance behind this edge, good efficiency can be ensured even for small signals.  (This method gives answers in close agreement with a point 20% of the way along the plateau for a module of typically 1450 ENC noise.)

### Method

The macro first performs a threshold scan with 2fC injected charge and set the threshold of each chip in accordance with the calculated mean VT50 values. Next the actual delay scan is performed as described below:

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | ON |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | 01X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | None |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | (2.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | 40mV (4.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Calibration Pulse + 131 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – Delay | 0 to 63, step size = 1 |  |  |  |  |  |  |  |  |  |  |  |
|  | N Triggers | 1000 |  |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

The analysis as described below is repeated for each chip in turn.

An error function is fitted to the rising edge of a strobe delay peak, corresponding to the falling edge of the signal.  Similarly a complementary error function is fitted to the falling edge, corresponding to the rising edge of the signal.  The width of the strobe delay peak is now known and the strobe delay register is set to a point 25% of the distance between the two edges, such that the electronics remain efficient for smaller signals.

If the fit to the rising edge of the strobe delay peak returns a value below the permitted minimum of 0 or the fit to the falling edge returns a value above the permitted minimum of 35, the defect SD_LO is recorded.  Similarly if the fit to the falling edge of the strobe delay peak returns a value above the permitted maximum of 28 or the fit to the falling edge returns a value above the permitted maximum of 63, the defect SD_HI is recorded.

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTHYBDELAY:	Strobe delay setting for each chip

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| SD_LO | Chip * 128 | ((Chip+1) * 128) -1 |
| SD_HI | Chip * 128 | ((Chip+1) * 128) -1 |

RAW DATA:

ABCD_CONFIG (optional)

### Acceptance

None of the defects found by this test are allowable on production modules, hence the “PASS” field will be set to “NO” if any defects were found.

The database reporting structure should also provide the possibility to set acceptance cuts on the range of delay values.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-11

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:17:13”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%StrobeDelay

#

#DELAY

#M0 S1  S2  S3  S4  E5

-1  –1  –1  –1  16  17

#

#M8 S9  S10 S11 S12 E13

17  15  16  16  16  18

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 0

LAST CHANNEL	: 127

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 128

LAST CHANNEL	: 255

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 256

LAST CHANNEL	: 383

#

%Defect

DEFECT NAME	: SD_LO

FIRST CHANNEL	: 384

LAST CHANNEL	: 511

#

#4 defects found

#

#No raw data!

## ThreePointGain.cpp

Threshold scans are taken for three injected charges to facilitate a quick measurement of gain and noise and to give an estimation of the discriminator offset.   Notably in the case of a module, threshold scans taken with small charges (<1.5fC) yield s-curves which are distorted at low thresholds due to noise occupancy.  Fitting the complementary error function to such an s-curve, the noise is under estimated.  For this reason, it is best to make noise measurements with injected charges of 1.5fC or more.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | OFF |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | X1X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | None |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | (variable) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | (trimmed or untrimmed as appropriate) |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Calibration Pulse + 131 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – Qinj | 1.50fC, 2.00fC, 2.50fC |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 – THRESHOLD | Ranges and step sizes to be determined |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 – CAL LINE | 0, 1, 2, 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | 1000 |  |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

A complementary error function is fitted to each threshold scan to yield values of VT50 and output noise for each channel.  A straight line is fitted to each set of three VT50 points to determine the gain and offset of each channel.  The input noise can now be calculated by dividing the output noise measured at 2fC by the calculated gain.

Anomalous channels are categorised as follows:

| Defect | Condition | #### Default value of cut |
| --- | --- | --- |
| DEAD | No output | - |
| STUCK | Continuous output | - |
| LO_GAIN | Gain < MIN_GAIN | 0 (mV/fC) |
| HI_GAIN | Gain > MAX_GAIN | 100 (mV/fC) |
| LO_OFFSET | Offset < MIN_OFFSET | -100 (mV) |
| HI_OFFSET | Offset > MAX_OFFSET | 120 (mV) |
| UNBONDED | Noise < BONDED_NOISE | 750 (ENC) |
| PARTBONDED | Noise < MIN_INNSE | 1100 (ENC) |
| NOISY | Noise > (mean_noise + (3* rms_noise)) | - |
| INEFFICIENT | Maximum efficiency <100% | (not yet implemented) |

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTSCANINFO:	number of scan points and corresponding charge values

TSTHYBRC:		response curve, gain, noise and offset information

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| DEAD | First Channel | Last Channel |
| STUCK | First Channel | Last Channel |
| LO_GAIN | First Channel | Last Channel |
| HI_GAIN | First Channel | Last Channel |
| LO_OFFSET | First Channel | Last Channel |
| HI_OFFSET | First Channel | Last Channel |
| UNBONDED | First Channel | Last Channel |
| PARTBONDED | First Channel | Last Channel |
| NOISY | First Channel | Last Channel |
| INEFF | First Channel | Last Channel |

RAW DATA:

ABCD_CONFIG (optional)

CHANNEL_DATA (mandatory)

### Acceptance

Use of the “PASS” field has not been implemented for this test, it is always set to be “YES”.

The database reporting structure must provide the possibility to set acceptance cuts on the mean and rms of VT50, gain, offset and noise, together with cuts on the maximum number of allowable channel defects falling into each of the following two categories:

A) LOST	DEAD, STUCK, UNBONDED, NOISY

B) FAULTY	LO_GAIN, HI_GAIN, LO_OFFSET, HI_OFFSET,

PARTBONDED, INEFFICIENT

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-12

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:19:21”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%SCAN_INFO

#

#POINT TYPE

“QCAL (fC)”

#N_POINTS

3

#POINTS

1.50	2.00	2.50

#

%ThreePointGain

#

#Loop A - Fit

#	func	p0	p1	p2

#M0

4	0.00	0.00	0.00

#S1

4	0.00	0.00	0.00

#S2

4	0.00	0.00	0.00

#S3

4	0.00	0.00	0.00

#S4

4	6.86	59.97	0.00

#E5

4	6.65	60.45	0.00

#M8

4	12.97	55.31	0.00

#S9

4	11.82	55.19	0.00

#S10

4	9.11	57.38	0.00

#S11

4	8.23	53.75	0.00

#S12

4	11.84	55.00	0.00

#E13

4	10.72	60.70	0.00

#

#Loop B - Gain, Offset, Noise at 2.00fC

#	vt50	rms	gain	rms	offset	rms	outnse	innse	rms

#M0

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S1

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S2

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S3

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S4

127.2	 8.58	60.0	1.40	  9.6	 6.02	14.00	 1460	  29

#S5

128.7	 7.55	60.4	1.36	  8.1	 4.95	12.90	 1336	  32

#M8

123.8	 8.64	55.3	1.21	 13.9	 7.60	13.68	 1546	  79

#S9

122.5	10.16	55.2	1.04	 14.4	 7.49	13.49	 1528	  40

#S10

124.6	10.09	57.4	1.24	 12.0	 7.67	13.71	 1494	  34

#S11

115.9	 9.68	53.8	1.13	 10.8	 7.36	12.93	 1504	  35

#S12

121.8	 9.32	55.0	1.24	 12.8	 7.77	13.31	 1513	 116

#E13

132.5	 7.63	60.7	1.04	 11.8	 6.29	13.59	 1400	  34

#

#Loop C – comment

#M0	S1	S2	S3	S4	E5

“dead”	“dead”	“dead”	“dead”	“OK”	“OK”

#M8	S9	S10	S11	S12	E13

“OK”	“OK”	“OK”	“OK”	“OK”	“OK”

#

%Defect

DEFECT NAME	: DEAD

FIRST CHANNEL	: 0

LAST CHANNEL	: 511

#

%Defect

DEFECT NAME	: PARTBONDED

FIRST CHANNEL	: 768

LAST CHANNEL	: 768

#

%Defect

DEFECT NAME	: PARTBONDED

FIRST CHANNEL	: 774

LAST CHANNEL	: 774

#

%Defect

DEFECT NAME	: UNBONDED

FIRST CHANNEL	: 1391

LAST CHANNEL	: 1391

#

%Defect

DEFECT NAME	: UNBONDED

FIRST CHANNEL	: 1393

LAST CHANNEL	: 1393

#

#5 defects found

#

%TEST Rawdata

FILENAME	: D:\sctvar\results\20220170100016_rc_246-12.txt

## TrimRange.cpp

During production, the chips used to build modules will be selected such that the offsets of all channels may be brought into line by adjustment of the TrimDAC using the minimum step size (TrimRange).  In ATLAS it will be necessary to increase the size of each TrimDAC in order to accommodate the increase in the offset spread as a function of radiation dose.  Hence it is necessary to determine the initial TrimDAC characteristic for each of the four TrimRange settings.

For TrimRange 0, the TrimDAC characteristic is studied using each of the possible TrimDAC values.  This is done to ensure that each step may be selected and to demonstrate that the TrimDAC characteristic is linear and monotonic for all steps.  For the remaining three TrimRange settings, the TrimDAC characteristic is studied using only 4 of the possible TrimDAC values.  The prime motivation for this reduction in the number of component scans is to save time during testing.

Since the nominal operating threshold in ATLAS will be 1.0fC, the TrimDAC/TrimRange characterisation should be performed in response to 1.0fC injected charge.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | OFF |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | X1X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | None |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | 10mV (1.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Calibration Pulse + 131 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – Trim Range | 0 to 4, step size = 1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 – TrimDAC | TR=0:(0 to 15) TR=1,2,3: (3, 7, 11, 15) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 – THRESHOLD | Ranges and step sizes to be determined |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 – CAL LINE | 0, 1, 2, 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | 200 |  |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

Each scan is fitted in turn to yield a matrix giving the threshold needed to obtain 50% efficiency (VT50) for 1fC input charge as a function of TrimDAC setting.  For each combination of TrimRange and channel number, a straight line is fitted to the data to determine the step size and offset of the corresponding TrimDAC characteristic.  Any points for which the s-curve fitting returned an error are ignored but the present version of the code does not include a noise cut at this stage.

For each TrimRange, a range of targets (2.5mV to 300mV in 2.5mV steps) is studied to determine the maximum number of channels that can be brought into line.  For each channel in turn, the fitted TrimDAC characteristic is solved to determine the TrimDAC setting needed to reach the studied targets.  If the calculated value is within the range of possible TrimDAC settings, the channel is considered to be trimmable for that combination of TrimRange and target.

At a second pass, the TrimRange value for each chip is chosen to maximise the number of trimmable channels.  By default this is done such that all channels of the module are trimmed to the same target value, but the option exists to allow each chip to be trimmed to a different target.

For each TrimRange setting, an ASCII file is generated listing the TrimDAC values for each chip.  For historical reasons this takes the form of a floating point number between 0 and 1, but this may change to a more logical format such as an integer between 0 and 15.  A fifth trim file is generated listing the TrimRange and TrimDAC settings found to be best overall for the hybrid or module under test.

In each of the above five cases a corresponding mask file is generated listing those channels to be masked and the reasons why.  In addition to untrimmable channels, dead, stuck and particularly noisy channels are also masked.  (In this instance a particularly noisy channel is defined to be one with output noise more than 125% the mean output noise of the parent chip.)

Defective chips and/or channels are categorised as follows:

| Defect | Description | #### Condition(s) |
| --- | --- | --- |
| TR_RANGE | A chip for which the mean step size is not as expected for the presently selected TrimRange | TR0: step < 1.5 or step > 5.0 TR1: step < 5.0 or step > 8.5 TR2: step < 8.5 or step > 12.0 TR3: step < 12.0 or step > 15.5 |
| TR_STEP | A channel for which the step size differs from the mean step size of that chip | Step < (mn_step - (3* rms_step)) |
|  |  | Step > (mn_step + (3* rms_step)) |
| TR_OFFSET | A channel for which the offset differs from the mean offset of that chip | Offset < (mn_offset - (3* rms_offset)) |
|  |  | Offset > (mn_offset + (3* rms_offset)) |

### Database

The results of this test should be stored in the SCT database using five instances of the following table, with TRIM_TYPE = {0,1,2,3,-1} corresponding to TrimRange=0, TrimRange=1, TrimRange=2, TrimRange=3, and with optimised TrimRange settings.  In each case the trim and mask files should be uploaded to the database for future reference.

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTSCANINFO:	number of scans and corresponding TrimDAC settings

TSTHYBTRIM:		trim summary

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| TR_RANGE | Chip * 128 | ((Chip+1) *128)-1 |
| TR_STEP | First Channel | Last Channel |
| TR_OFFSET | First Channel | Last Channel |

RAW DATA:

ABCD_CONFIG (optional)

TRIM_DATA (mandatory)

### Acceptance

##### The “PASS” field is set to “NO” if any defect of type TR_RANGE has been recorded.

The database reporting structure should also provide the possibility to set acceptance cuts on the number of trimmable channels, the mean and rms of the distribution of vt50 values after trimming, the TrimDAC offset and the TrimDAC step size.

### Sample Output from the Results File

Example 1: with all chips set to TrimRange 0

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-15

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:52:39”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%SCAN_INFO

%

#POINT TYPE

“TrimDAC (bits)”

#N_POINTS

16

#POINTS

0	1	2	3	4	5	6	7

8	9	10	11	12	13	14	15

#

%Trim

#

#TRIM	CHARGE	TYPE	ALGORITHM

10.0	0	0

#(trim whole module)

#

#	range	target	ntrim	vt50	vt50rms	tr_off	off_rms	tr_step	step_rms

#M0

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S1

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S2

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S3

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S4

0	87.5	128	 86.8	 1.18	-20.2	 2.5	 3.33	 0.15

#E5

0	87.5	128	 86.9	 1.23	-19.1	 1.9	 3.62	 0.20

#M8

0	87.5	128	 87.1	 1.05	-19.6	 2.4	 3.47	 0.17

#S9

0	87.5	128	 86.9	 1.12	-19.3	 2.8	 3.48	 0.16

#S10

0	87.5	128	 87.1	 1.14	-22.6	 3.3	 2.99	 0.15

#S11

0	87.5	128	 86.9	 1.05	-19.1	 3.0	 3.24	 0.19

#S12

0	87.5	128	 87.1	 1.10	-20.4	 2.7	 3.27	 0.21

#E13

0	87.5	127	 86.9	 1.27	-19.0	 2.1	 3.79	 0.20

#

%DEFECTS

#name	channel	value

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 0

LAST CHANNEL	: 127

#

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 128

LAST CHANNEL	: 255

#

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 256

LAST CHANNEL	: 383

#

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 384

LAST CHANNEL	: 512

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 636

LAST CHANNEL	: 636

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 639

LAST CHANNEL	: 639

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 895

LAST CHANNEL	: 895

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 1019

LAST CHANNEL	: 1019

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 1151

LAST CHANNEL	: 1151

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 721

LAST CHANNEL	: 721

#

#10 defects found

#

%TEST Rawdata

FILENAME	D:\sctvar\results\20220170100016_tr0_20010824.trim

Example 2: with optimised TrimRange settings

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-15

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:52:39”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%Trim

#

#TRIM	CHARGE	TYPE	ALGORITHM

10.0	-1	0

#(trim whole module)

#

TRIM_FILE	D:\sctvar\results\20220170100016_tr-1_20010824.trim

#	range	target	ntrim	vt50	vt50rms	tr_off	off_rms	tr_step	step_rms

#M0

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S1

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S2

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S3

0	87.5	0	  0.0	 0.00	 0.0	 0.0	 0.00	 0.00

#S4

87.5	128	 86.9	 0.96	-20.2	 2.5	 3.33	 0.15

#E5

87.5	128	 87.1	 1.03	-19.1	 1.9	 3.62	 0.20

#M8

87.5	128	 87.2	 0.92	-19.6	 2.4	 3.47	 0.17

#S9

87.5	128	 87.0	 0.97	-19.3	 2.8	 3.48	 0.16

#S10

7.5	128	 87.1	 1.00	-22.6	 3.3	 2.99	 0.15

#S11

7.5	128	 86.8	 0.96	-19.1	 3.0	 3.24	 0.19

#S12

7.5	128	 87.1	 0.97	-20.4	 2.7	 3.27	 0.21

#E13

7.5	128	 87.2	 2.22	-9.7	 1.1	 7.37	 0.36

#

%DEFECTS

#name	channel	value

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 0

LAST CHANNEL	: 127

#

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 128

LAST CHANNEL	: 255

#

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 256

LAST CHANNEL	: 383

#

%Defect

DEFECT NAME	: TR_RANGE

FIRST CHANNEL	: 384

LAST CHANNEL	: 512

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 636

LAST CHANNEL	: 636

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 639

LAST CHANNEL	: 639

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 895

LAST CHANNEL	: 895

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 1019

LAST CHANNEL	: 1019

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 1151

LAST CHANNEL	: 1151

#

%Defect

DEFECT NAME	: TR_STEP

FIRST CHANNEL	: 721

LAST CHANNEL	: 721

#

#10 defects found

#

%TEST Rawdata

FILENAME	D:\sctvar\results\20220170100016_tr-1_20010824.trim

## ResponseCurve.cpp

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | OFF |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | X1X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | Untrimmable, Stuck, Dead and Noisy Channels |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | (variable) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Calibration Pulse + 131 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – Qinj | 0.50fC, 0.75fC, 1.00fC, 1.25fC, 1.50fC, 2.00fC, 3.00fC, 4.00fC, 6.00fC, 8.00fC |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 – THRESHOLD | Ranges and step sizes to be determined |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 – CAL LINE | 0, 1, 2, 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | 1000 |  |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

A complementary error function is fitted to each threshold scan to yield values of VT50 and output noise for each channel.  A straight line is fitted to each set of three VT50 points to determine the gain and offset of each channel.  The input noise can now be calculated by dividing the output noise measured at 2fC by the calculated gain.

Anomalous channels are categorised as follows:

| Defect | Condition | #### Default value of cut |
| --- | --- | --- |
| DEAD | No output | - |
| STUCK | Continuous output | - |
| LO_GAIN | Gain < MIN_GAIN | 0 (mV/fC) |
| HI_GAIN | Gain > MAX_GAIN | 100 (mV/fC) |
| LO_OFFSET | Offset < MIN_OFFSET | -100 (mV) |
| HI_OFFSET | Offset > MAX_OFFSET | 120 (mV) |
| UNBONDED | Noise < BONDED_NOISE | 750 (ENC) |
| PARTBONDED | Noise < MIN_INNSE | 1100 (ENC) |
| NOISY | Noise > (mean_noise + (3* rms_noise)) | - |
| INEFFICIENT | Maximum efficiency <100% | (not yet implemented) |

### Database

This test has the same inputs and outputs as ThreePointGain.cpp:

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTSCANINFO:	number of scan points and corresponding charge values

TSTHYBRC:		response curve, gain, noise and offset information

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| DEAD | First Channel | Last Channel |
| STUCK | First Channel | Last Channel |
| LO_GAIN | First Channel | Last Channel |
| HI_GAIN | First Channel | Last Channel |
| LO_OFFSET | First Channel | Last Channel |
| HI_OFFSET | First Channel | Last Channel |
| UNBONDED | First Channel | Last Channel |
| PARTBONDED | First Channel | Last Channel |
| NOISY | First Channel | Last Channel |
| INEFF | First Channel | Last Channel |

RAW DATA:

ABCD_CONFIG (optional)

CHANNEL_DATA (mandatory)

### Acceptance

Use of the “PASS” field has not been implemented for this test, it is always set to be “YES”.

The database reporting structure must provide the possibility to set acceptance cuts on the mean and rms of VT50, gain, offset and noise, together with cuts on the maximum number of allowable channel defects falling into each of the following two categories:

A) LOST	DEAD, STUCK, UNBONDED, NOISY

B) FAULTY	LO_GAIN, HI_GAIN, LO_OFFSET, HI_OFFSET,

PARTBONDED, INEFFICIENT

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-43

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

”18:41:55”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%SCAN_INFO

#

#POINT TYPE

“QCAL (fC)”

#N_POINTS

10

#POINTS

0.50	0.75	1.00	1.25	1.50	2.00	3.00	4.00

6.00	8.00

#

%ResponseCurve

#

#Loop A - Fit

#	func	p0	p1	p2

#M0

3	0.01	46.01	-0.00

#S1

3	0.01	46.01	-0.00

#S2

3	0.01	46.01	-0.00

#S3

3	0.01	46.01	-0.00

#S4

3	1571.96	6.63	-757.75

#E5

3	1572.40	6.71	-757.43

#M8

3	1676.52	7.59	-805.92

#S9

3	1613.51	7.35	-774.31

#S10

3	1659.59	7.37	-798.54

#S11

3	1802.85	8.44	-867.39

#S12

3	1716.86	7.78	-826.15

#E13

3	1637.46	6.78	-791.18

#

#Loop B - Gain, Offset, Noise at 2.00fC

#	vt50	rms	gain	rms	offset	rms	outnse	innse	rms

#M0

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S1

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S2

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S3

0.0	 0.00	 0.0	0.00	  0.0	 0.00	0.00	    0	   0

#S4

146.6	 1.80	57.8	1.42	 28.6	 1.91	13.82	 1494	  34

#E5

146.9	 1.72	57.3	1.05	 28.9	 1.54	13.23	 1443	  35

#M8

142.1	 1.62	54.2	1.25	 32.5	 1.69	13.39	 1545	  76

#S9

141.7	 1.38	53.7	1.11	 32.8	 1.83	13.33	 1553	  40

#S10

144.0	 1.50	55.2	1.20	 31.4	 1.71	13.70	 1550	  34

#S11

140.3	 1.42	52.4	1.10	 34.4	 1.73	12.97	 1546	  30

#S12

141.6	 1.49	54.0	1.24	 32.7	 1.92	13.40	 1551	 120

#E13

147.9	 2.51	58.9	0.93	 27.9	 2.66	13.77	 1463	  37

#

#Loop C – comment

#M0	S1	S2	S3	S4	E5

“dead”	“dead”	“dead”	“dead”	“OK”	“OK”

#M8	S9	S10	S11	S12	E13

“OK”	“OK”	“OK”	“OK”	“OK”	“OK”

#

%Defect

DEFECT NAME	: DEAD

FIRST CHANNEL	: 0

LAST CHANNEL	: 511

#

%Defect

DEFECT NAME	: PARTBONDED

FIRST CHANNEL	: 768

LAST CHANNEL	: 768

#

%Defect

DEFECT NAME	: PARTBONDED

FIRST CHANNEL	: 774

LAST CHANNEL	: 774

#

%Defect

DEFECT NAME	: UNBONDED

FIRST CHANNEL	: 1391

LAST CHANNEL	: 1391

#

%Defect

DEFECT NAME	: UNBONDED

FIRST CHANNEL	: 1393

LAST CHANNEL	: 1393

#

#5 defects found

#

%TEST Rawdata

FILENAME	: D:\sctvar\results\20220170100016_rc_246-43.txt

## NO.cpp

For this measurement, the trigger frequency should approach the nominal ATLAS trigger rate of 100kHz, being throttled back as necessary at higher occupancies in order to avoid buffer overflow errors.  In order to measure noise occupancies down to the level of 10-6 up to 106 events are taken.  At the other end of the scale, where occupancy approaches 1.0, only 2000 events are recorded.  Between the two extremes the fractional occupancy of each channel is calculated after each partial burst and the number of events taken is varied such that, for each scanpoint, a minimum of 50 hits are seen in more than 50% of the active readout channels.  In the case that the occupancy of a given channel should overflow the depth of the MuSTARD histogram memory, its fractional occupancy is frozen as that determined prior to the overflow condition.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | OFF |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | X1X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | Untrimmable, Stuck, Dead and Noisy Channels |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | - |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | Up to 100kHz, depending upon occupancy |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – THRESHOLD | 0 – 150mV in 2.5mV steps (typically to cover the range –1.0fC to +2.0fC) |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | Up to 1000,000, depending upon occupancy |  |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

This scan yields a direct measurement of noise occupancy at 1fC. The trim information is read in from the most recent results file such that the 1fC point is accurately known: in this the module currents and its noise occupancy at the nominal operating point can be extracted from the data.

By fitting an error function to the noise occupancy curve for each chip, the noise occupancy offset is determined. The response curve is read in from the most recent results file and the information is used to fit a straight line to a plot of log occupancy vs. threshold2 (fC2) to yield a measurement of the noise.

Defective channels are categorised as follows:

| Defect | Description | #### Condition(s) |
| --- | --- | --- |
| NO_HI | A channel with high noise occupancy | Occupancy > 5x10-4 |

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTHYBNOISE:	noise occupancy at 1fC, NO offset and estimated noise (ENC)

DEFECTS:

| Defect | Channel | Last Channel |
| --- | --- | --- |
| NO_HI | First Channel | Last Channel |

### Acceptance

##### Use of the “PASS” field is not yet implemented for this test, hence it is always set to “YES”.

The database reporting structure should provide the possibility to set acceptance cuts on each of the following parameters:

- The extrapolated noise occupancy offset,

- The mean noise occupancy at 1fC

- RMS of the noise occupancy at 1fC

- The estimated noise at 1fC

- The number of channels with low noise occupancy

- The number of channels with high noise occupancy

- The typical and maximum values of Icc and Idd.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-53

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:47:33”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%NO

#

#MAX	ICC	IDD

- 710

#TYP	ICC	IDD

- 535

#

#	Offset	MeanOcc   	RMSOcc    	EstENC

#M0

0.0	0.0e+000	0.0e+000	   0

#S1

0.0	0.0e+000	0.0e+000	   0

#S2

0.0	0.0e+000	0.0e+000	   0

#S3

0.0	0.0e+000	0.0e+000	   0

#S4

32.9	1.6e-005	8.6e-006	1407

#E5

31.0	1.1e-006	1.2e-006	1291

#M8

30.6	1.3e-005	5.6e-006	1458

#S9

31.5	9.6e-006	6.4e-006	1430

#S10

31.3	1.1e-005	5.4e-006	1420

#S11

31.8	7.1e-006	3.7e-006	1412

#S12

30.5	8.7e-006	5.0e-006	1416

#E13

26.4	2.7e-006	3.5e-006	1343

#

#No defects found!

#

#No raw data!

## Timewalk.cpp

In the ABCD3T Chip specification document, timewalk is defined as, “The maximum time variation in the crossing of the time stamp threshold over a signal range of 1.25 to 10.0fC, with the comparator set to 1fC”.  This procedure sets out to make this measurement.

### Method

| SETTINGS | ##### Configuration | Master/Slave/End | M | S | S | S | S | E | M | S | S | S | S | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge Detect | ON |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Data Compression | 01X |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mode | Data Taking Mode |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Select | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Masked Channels | Untrimmable, Stuck, Dead and Noisy Channels |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### DACs | Ipreamp | 220A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ishaper | 30A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Threshold DAC | (1fC) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Calibration DAC | (variable) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Delay Register | (scanned) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimRange | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TrimDAC | (optimised) |  |  |  |  |  |  |  |  |  |  |  |
|  | ##### TRIGGER | Sequence | Calibration Pulse + 131 BCO delay + L1A |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Frequency | - |  |  |  |  |  |  |  |  |  |  |  |
| SCANS | ##### LOOPS | 1 – Qinj | 1.25fC, 10.0fC |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 – Delay | 0 to 63, step size = 1 |  |  |  |  |  |  |  |  |  |  |  |
|  | n Triggers | 1000 |  |  |  |  |  |  |  |  |  |  |  |  |

### Analysis

For each value of injected charge, a complementary error function is fitted to the falling edge of a plot of efficiency vs. the setting of the delay register, to determine the delay setting at which the efficiency reaches half of its peak value.  The difference between these two values gives the timewalk in terms of steps on the delay register.

In the case of the delay scan taken with 10fC injected charge, an error function is fitted to the rising edge of the efficiency plot.  Since these scans are taken in edge mode the width of the strobe delay peak will be 25nS.  This provides a calibration factor for the strobe delay register, hence the timewalk can be converted to units of nS.

Anomalous timewalk values are categorised as defects as follows:

| Defect | Condition | #### Default value of cut |
| --- | --- | --- |
| TW_LO | Timewalk < MIN_TW | 5 |
| TW_HI | Timewalk > MAX_TW | 16 |

### Database

The following information should be stored in the SCT database:

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures

TSTSCANINFO:	number of scans and corresponding charge values

TSTHYBTW:		timewalk (nS) and calibration factor of each chip

DEFECTS:

| Defect | First Channel | Last Channel |
| --- | --- | --- |
| TW_LO | First Channel | Last Channel |
| TW_HI | First Channel | Last Channel |

### Acceptance

The “PASS” field will be set to “NO” if any defects are found.

The database reporting structure should provide the possibility to set more stringent acceptance cuts on the calculated value of the timewalk and the strobe delay calibration factor.

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: pwp

LOCATION NAME	: RAL

Run number	: 256-55

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“HEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“18:58:22”

#

%DCS_INFO

#

#T0	T1

25.5	26.3

#VDET	IDET

350	1.220

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%SCAN_INFO

#

#POINT TYPE

“QCAL (fC)”

#N_POINTS

10

#POINTS

1.25	1.50	1.75	2.00	3.00	4.00	5.00	6.00

8.00	10.00

#

%Timewalk

#

#TW

#M0  S1   S2   S3   S4   E5

0.0  0.0  0.0  0.0  11.9 11.4

#

#M8  S9   S10  S11  S12  E13

11.4 11.6 11.9 11.7 11.4 10.7

#

#TCAL

#M0  S1   S2   S3   S4   E5

-1.0 –1.0 –1.0 –1.0 28.9 29.5

#

#M8  S9   S10  S11  S12  E13

28.9 28.5 29.6 28.2 29.4 30.7

#

%Defect

DEFECT NAME	: TW_LO

FIRST CHANNEL	: 0

LAST CHANNEL	: 127

#

%Defect

DEFECT NAME	: TW_LO

FIRST CHANNEL	: 128

LAST CHANNEL	: 255

#

%Defect

DEFECT NAME	: TW_LO

FIRST CHANNEL	: 256

LAST CHANNEL	: 383

#

%Defect

DEFECT NAME	: TW_LO

FIRST CHANNEL	: 384

LAST CHANNEL	: 511

#

#4 defects found

#

#No raw data!

# Description of Other Electrical Tests

## HybridLTT.cpp

### Method

For the test duration of 100 hours, a hybrid is maintained such that its thermistors report a constant temperature in the region of 37OC.  Every two hours a confirmation test is executed, and the list of defects is compared to those which are already known.  Any newly found defects are logged in the database.  A characterisation run is initiated at the end of the run.

### Analysis

The results of each confirmation sequence will be compared against earlier data to identify the occurrence of any new defects.

### Database

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures at the end of test

TSTHYBLTT:		minimum and maximum temperatures and currents during the test

DEFECTS:		any defect type appropriate to confirmation sequence, but only newly

occurring ones are recorded.

RAW DATA:

ABCD_CONFIG (optional)

LTT_MONITOR_DATA (mandatory)

### Sample Output from the Results File

#

%NewTest

#

SERIAL NUMBER	: 20220170100016

TEST MADE BY	: dgc

LOCATION NAME	: Birmingham

Run number	: 123-1

TEST_DATE	: 09/08/2001

PASSED		: YES

PROBLEM		: NO

#

%DAQ_INFO

#

#HOST

“NOTHEPNTW124”

#VERSION

“3.10”

#DUT

“Barrel Module”

#TIME

“10:00:00”

#

%DCS_INFO

#

#T0	T1

37.5	38.3

#VDET	IDET

0	0.000

#VCC	ICC

3.500	790

#VDD	IDD

4.000	535

#TIME_POWERED

.

#

%LongTermTest

#

#Duration

99.9

#Time of first failure

2.0

#T0 min max

36.4	37.5

#T1 min max

36.8	38.3

#Icc min max

780	800

#Idd min max

- 540

#Idet min max

. 	.

#

%Defect

DEFECT NAME	: DEAD

FIRST CHANNEL	: 128

LAST CHANNEL	: 255

URL		: 10.59.23 09/08/2001

#

#1 new defect found

#

%TEST Rawdata

FILENAME	: D:\sctvar\results\20220170100016_ltt_246-1.txt

## ModuleLTT.cpp

### Method

For the test duration of 24 hours, a module is maintained at the operating temperature expected in ATLAS.  Every two hours a confirmation test is executed, and the list of defects is compared to those which are already known.  Any newly found defects are logged in the database.  A characterisation run is initiated at the end of the run.

### Analysis

The results of each confirmation sequence will be compared against earlier data to identify the occurrence of any new defects.

### Database

TSTDAQINFO:		host and version information

TSTDCSINFO:		monitored voltages, currents and temperatures at the end of test

TSTHYBLTT:		minimum and maximum temperatures and currents during the test

DEFECTS:		any defect type appropriate to confirmation sequence, but only newly

occurring ones are recorded.

RAW DATA:

ABCD_CONFIG (optional)

LTT_MONITOR_DATA (mandatory)

## Appendices

## Appendix 1 – Definitions of Utility Tables

TSTDAQINFO: 	tag “%DAQ_INFO”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| HOST | O | Char(30) | - | Hostname of test PC |
| VERSION | O | Char(10) | - | Software Version, e.g. 3.10 |
| DUT | O | Char(20) | - | Device Under Test eg. “Barrel Hybrid” or “Barrel Module” |
| TEST_TIME | O | Char(10) | - | Time the test started |

TSTDCSINFO:		tag “%DCS_INFO”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| T0 | O | Float | -100 to 200 | Module Temp. 0 (OC) |
| T1 | O | Float | -100 to 200 | Module Temp. 1 (OC) |
| Vdet | O | Float | 0 to 500 | Detector Voltage (V) |
| Idet | O | Float | 0 to 5200 | Detector Current (A) |
| Vcc | O | Float | 0 to 10 | Analogue Voltage (V) |
| Icc | O | Float | 0 to 10 | Analogue Current (A) |
| Vdd | O | Float | 0 to 10 | Digital Voltage (V) |
| Idd | O | Float | 0 to 10 | Digital Current (A) |
| TIME_POWERED | O | Float | 0 – inf. | (hours) |

TSTSCANINFO:	tag “%SCAN_INFO”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| POINT_TYPE | M | Char(20) | - | Scan point descriptor |
| N_POINTS | M | Int (8 bit) | 0 to 16 | Number of scans |
| POINT_0 | O | Float | -inf to inf | Scan point setting |
| POINT_1 | O | Float | -inf to inf | Scan point setting |
| POINT_2 | O | Float | -inf to inf | Scan point setting |
| POINT_3 | O | Float | -inf to inf | Scan point setting |
| POINT_4 | O | Float | -inf to inf | Scan point setting |
| POINT_5 | O | Float | -inf to inf | Scan point setting |
| POINT_6 | O | Float | -inf to inf | Scan point setting |
| POINT_7 | O | Float | -inf to inf | Scan point setting |
| POINT_8 | O | Float | -inf to inf | Scan point setting |
| POINT_9 | O | Float | -inf to inf | Scan point setting |
| POINT_10 | O | Float | -inf to inf | Scan point setting |
| POINT_11 | O | Float | -inf to inf | Scan point setting |
| POINT_12 | O | Float | -inf to inf | Scan point setting |
| POINT_13 | O | Float | -inf to inf | Scan point setting |
| POINT_14 | O | Float | -inf to inf | Scan point setting |
| POINT_15 | O | Float | -inf to inf | Scan point setting |

## Appendix 2 – Definitions of Result Tables

TSTMODIV:		tag “%ModIVScan”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| TEMPERATURE | M | Float |  | For compatibility with existing IV table (=T0) |
| I_LEAK_150 | M | Float | 0 to 5200 | Leakage Current (A) |
| I_LEAK_350 | M | Float | 0 to 5200 | Leakage Current (A) |

TSTHYBRESET:	tag “%HardReset”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| ICC_NOCONFIG | M | Float | 0 to 2000 | Analogue Current (mA) |
| IDD_NOCONFIG | M | Float | 0 to 2000 | Digital Current (mA) |
| ICC_NOCLOCK | M | Float | 0 to 2000 | Analogue Current (mA) |
| IDD_NOCLOCK | M | Float | 0 to 2000 | Digital Current (mA) |

TSTHYBPIPE:		tag “%PipelineTest”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| M0_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S1_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S2_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S3_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S4_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| E5_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| M8_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S9_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S10_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S11_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| S12_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |
| E13_NGOOD | M | Int (8 bit) | 0 to 128 | No. good channels |

TSTHYBBPASS:	tag “%FullBypassTest”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| M0_TOKEN | M | Float | 0 to 10 V |  |
| M0_RTOKEN | M | Float | 0 to 10 V |  |
| S1_TOKEN | M | Float | 0 to 10 V |  |
| S1_RTOKEN | M | Float | 0 to 10 V |  |
| S2_TOKEN | M | Float | 0 to 10 V |  |
| S2_RTOKEN | M | Float | 0 to 10 V |  |
| S3_TOKEN | M | Float | 0 to 10 V |  |
| S3_RTOKEN | M | Float | 0 to 10 V |  |
| S4_TOKEN | M | Float | 0 to 10 V |  |
| S4_RTOKEN | M | Float | 0 to 10 V | FORWARD ONLY |
| E5_TOKEN | M | Float | 0 to 10 V | FORWARD ONLY |
| E5_RTOKEN | M | Float | 0 to 10 V |  |
| M8_TOKEN | M | Float | 0 to 10 V |  |
| M8_RTOKEN | M | Float | 0 to 10 V |  |
| S9_TOKEN | M | Float | 0 to 10 V |  |
| S9_RTOKEN | M | Float | 0 to 10 V |  |
| S10_TOKEN | M | Float | 0 to 10 V |  |
| S10_RTOKEN | M | Float | 0 to 10 V |  |
| S11_TOKEN | M | Float | 0 to 10 V |  |
| S11_RTOKEN | M | Float | 0 to 10 V |  |
| S12_TOKEN | M | Float | 0 to 10 V |  |
| S12_RTOKEN | M | Float | 0 to 10 V | FORWARD ONLY |
| E13_TOKEN | M | Float | 0 to 10 V | FORWARD ONLY |
| E13_RTOKEN | M | Float | 0 to 10 V |  |
| M0_COMMENT | M | Char(20) |  |  |
| M0_RCOMMENT | M | Char(20) |  |  |
| S1_COMMENT | M | Char(20) |  |  |
| S1_RCOMMENT | M | Char(20) |  |  |
| S2_COMMENT | M | Char(20) |  |  |
| S2_RCOMMENT | M | Char(20) |  |  |
| S3_COMMENT | M | Char(20) |  |  |
| S3_RCOMMENT | M | Char(20) |  |  |
| S4_COMMENT | M | Char(20) |  |  |
| S4_RCOMMENT | M | Char(20) |  | FORWARD ONLY |
| E5_COMMENT | M | Char(20) |  | FORWARD ONLY |
| E5_RCOMMENT | M | Char(20) |  |  |
| M8_COMMENT | M | Char(20) |  |  |
| M8_RCOMMENT | M | Char(20) |  |  |
| S9_COMMENT | M | Char(20) |  |  |
| S9_RCOMMENT | M | Char(20) |  |  |
| S10_COMMENT | M | Char(20) |  |  |
| S10_RCOMMENT | M | Char(20) |  |  |
| S11_COMMENT | M | Char(20) |  |  |
| S11_RCOMMENT | M | Char(20) |  |  |
| S12_COMMENT | M | Char(20) |  |  |
| S12_RCOMMENT | M | Char(20) |  | FORWARD ONLY |
| E13_COMMENT | M | Char(20) |  | FORWARD ONLY |
| E13_RCOMMENT | M | Char(20) |  |  |

TSTHYBDELAY:	tag “%StrobeDelay”

| Column Name |  | Representation |  | Comment |
| --- | --- | --- | --- | --- |
| M0_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S1_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S2_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S3_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S4_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| E5_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| M8_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S9_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S10_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S11_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| S12_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |
| E13_DELAY | M | Int (8 bit) | -1 to 63 | Optimum Strobe Delay |

TSTHYBRC:	tags “%ThreePointGain” and “%ResponseCurve”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| Loop A |  |  |  |  |
| A_RC_FIT | M | Float | 0 to 255 | Fit function type |
| A_RC_P0 | M | Float | -inf to inf | Fit parameter 0 |
| A_RC_P1 | M | Float | -inf to inf | Fit parameter 1 |
| A_RC_P2 | M | Float | -inf to inf | Fit parameter 2 |
| Where A ={M0,S1,S2,S3,S4,E5,M8,S9,S10,S11,S12,E13} |  |  |  |  |
| Loop B |  |  |  |  |
| B_RC_MN_VT50 | M | Float | 0 to 640 | Mean VT50 at 1fC (mV) |
| B_RC_RMS_VT50 | M | Float | 0 to 30 | RMS VT50 at 1fC (mV) |
| B_RC_MN_GAIN | M | Float | 0 to 100 | Mean gain at 2fC(mV/fC) |
| B_RC_RMS_GAIN | M | Float | 0 to 30 | RMS gain at 2fC (mV/fC) |
| B_RC_MN_OFFSET | M | Float | -100 to 120 | Extrapolated Offset(mV) |
| B_RC_RMS_OFFSET | M | Float | 0 to 30 | RMS Ext. Offset(mV) |
| B_RC_MN_NSE | M | Float | 0 to 120 | Mean output noise at 2fC (mV) |
| B_RC_MN_ENC | M | Float | 0 to 5000 | Mean input noise at 2fC (ENC) |
| B_RC_RMS_ENC | M | Float | 0 to 1000 | RMS input noise at 2fC (ENC) |
| Where B ={M0,S1,S2,S3,S4,E5,M8,S9,S10,S11,S12,E13} |  |  |  |  |
| Loop C |  |  |  |  |
| B_RC_COMMENT | M | Char(20) | - | Comment |
| Where C ={M0,S1,S2,S3,S4,E5,M8,S9,S10,S11,S12,E13} |  |  |  |  |

TSTHYBNOISE:	tag “%NO”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| ICC_MAX | M | Float | 0 to 2000 | Highest Icc (mA) |
| IDD_MAX | M | Float | 0 to 2000 | Highest Idd (mA) |
| ICC_TYP | M | Float | 0 to 2000 | Icc at 1fC (mA) |
| IDD_TYP | M | Float | 0 to 2000 | Idd at 1fC (mA) |
| Loop A |  |  |  |  |
| A_NO_OFFSET | M | Float | -100 to 120 | Noise occupancy offset (mV) |
| A_NO_MN_OCC | M | Float | 0 to 1 | Mean Noise occupancy at 1fC |
| A_NO_RMS_OCC | M | Float | 0 to 1 | RMS Noise occupancy at 1fC |
| A_NO_NSE | M | Float | 0 to 5000 | Estimated noise (ENC) |
| Where A ={M0,S1,S2,S3,S4,E5,M8,S9,S10,S11,S12,E13} |  |  |  |  |

TSTHYBTRIM:		tag “%Trim”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| TR_TYPE | M | Int (8 bit) | -1 to 127 | Trim Type (Range) |
| TR_ALGORITHM | M | Int (8 bit) | 0 to 255 | Trim Algorithm |
| Loop A |  |  |  |  |
| A_TR_RANGE | M | Float | 0 to 3 | Trim Range Setting |
| A_TR_TARGET | M | Float | 0 to 640 | Trim Target |
| A_TR_NTRIM | M | Floatxi | 0 to 128 | Number of trimmable channels |
| A_TR_MN_VT50 | M | Float | 0 to 640 | Mean VT50 after trimming (mV) |
| A_TR_RMS_VT50 | M | Float | 0 to 120 | RMS VT50 After trimming (mV) |
| A_TR_MN_STEP | M | Float | 0 to 20 | Mean TrimDAC step (mV) |
| A_TR_RMS_STEP | M | Float | 0 to 10 | RMS TrimDAC step (mV) |
| A_TR_MNOFFSET | M | Float | -120 to 120 | Mean TrimDAC Offset (mV) |
| A_TR_RMSOFFSET | M | Float | 0 to 10 | RMS TrimDAC offset (mV) |
| Where A ={M0,S1,S2,S3,S4,E5,M8,S9,S10,S11,S12,E13} |  |  |  |  |

TSTHYBTW:		tag “%Timewalk”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| M0_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S1_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S2_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S3_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S4_TW | M | Float | 0 to 25 | Timewalk (nS) |
| E5_TW | M | Float | 0 to 25 | Timewalk (nS) |
| M8_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S9_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S10_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S11_TW | M | Float | 0 to 25 | Timewalk (nS) |
| S12_TW | M | Float | 0 to 25 | Timewalk (nS) |
| E13_TW | M | Float | 0 to 25 | Timewalk (nS) |
| M0_TCAL | M | Float | -1 to 63 | Calibration factor |
| S1_TCAL | M | Float | -1 to 63 | Calibration factor |
| S2_TCAL | M | Float | -1 to 63 | Calibration factor |
| S3_TCAL | M | Float | -1 to 63 | Calibration factor |
| S4_TCAL | M | Float | -1 to 63 | Calibration factor |
| E5_TCAL | M | Float | -1 to 63 | Calibration factor |
| M8_TCAL | M | Float | -1 to 63 | Calibration factor |
| S9_TCAL | M | Float | -1 to 63 | Calibration factor |
| S10_TCAL | M | Float | -1 to 63 | Calibration factor |
| S11_TCAL | M | Float | -1 to 63 | Calibration factor |
| S12_TCAL | M | Float | -1 to 63 | Calibration factor |
| E13_TCAL | M | Float | -1 to 63 | Calibration factor |

TSTHYBLTT:		tag “%LongTermTest”

| Column Name |  | Representation | Range | Comment |
| --- | --- | --- | --- | --- |
| Duration | M | Float | 0 to 1000 | Duration of test (hours) |
| Fail_time | M | Float | -1 to 1000 | Time of first failure |
| T0_min | M | Float | -100 to 200 | Minimum hybrid temp. |
| T0_max | M | Float | -100 to 200 | Maximum hybrid temp. |
| T1_min | M | Float | -100 to 200 | Minimum hybrid temp. |
| T1_max | M | Float | -100 to 200 | Maximum hybrid temp. |
| Icc_min | M | Float | 0 to 2000 | Minimum analogue current |
| Icc_max | M | Float | 0 to 2000 | Maximum analogue current |
| Idd_min | M | Float | 0 to 2000 | Minimum digital current |
| Idd_max | M | Float | 0 to 2000 | Maximum digital current |
| Idet_min | O | Float | 0 to 5200 | Minimum detector current |
| Idet_max | O | Float | 0 to 5200 | Maximum detector current |

## Appendix 3 – List of Defect Names

| Defect | Associated Test(s) | Associated Table(s) | First Channel | Last Channel |
| --- | --- | --- | --- | --- |
| IV_LIMIT | IVCurve.cpp | TSTMODIV | 0 | 1535 |
| IV_TRIP |  |  | 0 | 1535 |
| HR_NOCLK | HardReset.cpp | TSTHYBRESET | Link * 768 | ((Link+1)*768)-1 |
| HR_NOCON |  |  | Link * 768 | ((Link+1)*768)-1 |
| HR_NORST |  |  | Link * 768 | ((Link+1)*768)-1 |
| CLK_ADDR0 | RedundancyTest.cpp | TSTHYBCLOCK | Chip * 128 | ((Chip+1)*128)-1 |
| CLK_ADDR1 |  |  | Chip * 128 | ((Chip+1)*128)-1 |
| CLK_COM0 |  |  | Chip * 128 | ((Chip+1)*128)-1 |
| CLK_COM1 |  |  | Chip * 128 | ((Chip+1)*128)-1 |
| CLK_ERROR |  |  | Chip * 128 | ((Chip+1)*128)-1 |
| TOKEN | FullBypassTest.cpp | TSTHYBBPASS | Chip * 128 | ((Chip+1)*128)-1 |
| RTOKEN |  |  | Chip * 128 | ((Chip+1)*128)-1 |
| DEAD | PipelineTest.cpp | TSTHYBPIPE | Channel | Channel |
| STUCK |  |  | Channel | Channel |
| DEADCELL |  |  | Channel | Channel |
| STUCKCELL |  |  | Channel | Channel |
| SD_LO | StrobeDelay.cpp | TSTHYBDELAY | Chip * 128 | ((Chip+1)*128)-1 |
| SD_HI |  |  | Chip * 128 | ((Chip+1)*128)-1 |
| DEAD | ThreePointGain.cpp ResponseCurve.cpp | TSTHYBRC | First Channel | Last Channel |
| STUCK |  |  | First Channel | Last Channel |
| LO_GAIN |  |  | First Channel | Last Channel |
| HI_GAIN |  |  | First Channel | Last Channel |
| LO_OFFSET |  |  | First Channel | Last Channel |
| HI_OFFSET |  |  | First Channel | Last Channel |
| UNBONDED |  |  | First Channel | Last Channel |
| PARTBONDED |  |  | First Channel | Last Channel |
| NOISY |  |  | First Channel | Last Channel |
| INEFF |  |  | First Channel | Last Channel |
| TR_RANGE | TrimRange.cpp | TSTHYBTRIM | Chip * 128 | ((Chip+1)*128)-1 |
| TR_STEP |  |  | First Channel | Last Channel |
| TR_OFFSET |  |  | First Channel | Last Channel |
| NO_HI | NO.cpp | TSTHYBNOISE | First Channel | Last Channel |
| TW_LO | Timewalk.cpp | TSTHYBTW | First Channel | Last Channel |
| TW_HI |  |  | First Channel | Last Channel |

## Appendix 4 – Database table implementation status

| Table | Table approved for addition to DB | Priority | Table added to the DB | File format approved for Java Application | Java Application Implemented |
| --- | --- | --- | --- | --- | --- |
| TSTDAQINFO | #### NOT YET | - | - | - | - |
| TSTDCSINFO | #### NOT YET | - | - | - | - |
| TSTSCANINFO | #### NOT YET | - | - | - | - |
| TSTMODIV | Already exists as TSTDETIV |  |  |  |  |
| TSTHYBRESET | NOT YET | - | - | - | - |
| TSTHYBCLOCK | NOT YET | - | - | - | - |
| TSTHYBBPASS | NOT YET | - | - | - | - |
| TSTHYBPIPE | NOT YET | - | - | - | - |
| TSTHYBDELAY | NOT YET | - | - | - | - |
| TSTHYBRC | NOT YET | - | - | - | - |
| TSTHYBTRIM | NOT YET | - | - | - | - |
| TSTHYBNOISE | NOT YET | - | - | - | - |
| TSTHYBTW | NOT YET | - | - | - | - |
| TSTHYBLTT | NOT YET | - | - | - | - |

# References

http://sct-testdaq.home.cern.ch/sct-testdaq/sctdaq/sctdaq.html

In order to upload the results to the database, one file for each test is needed rather than one file for each test sequence. A root macro has been written to split the results file into the appropriate number of segments needed for the upload procedure.  The files are deleted after upload.

The detector bias voltage used during tests of irradiated modules, subject to consideration of the detector leakage current, is typically 300V.

In the event that this test finds a channel to be dead, a fault in the mask register would be one possible cause.  In combination with the data set from the Redundancy Test it would be possible to identify dead cells in the mask register – but this seems excessive.

These cuts are provisional, and may change with future batches of ABCD3T wafers.

When running with EDGE DETECT ON / EDGE COMPRESSION, threshold scans with small charges do not reach 100% efficiency since the noise is of comparable value to the injected charge.  By running with edge detect off, in any other compression mode, such problems are minimised.

The 100kHz trigger rate and buffer overflow control have been implemented only for users of CLOAC.  A fallback test has been implemented for users with only SLOG, but the trigger rate is not controlled.

The defect identification mechanism is still under development, and is not yet included in any public release of SCTDAQ.

The defect identification mechanism is still under development, and is not yet included in any public release of SCTDAQ.

Not yet implemented

For reasons of convenience when reading the table into the database

For reasons of convenience when reading the table into the database