# Margins: T/B=2.2 cm, L/R=2.2 cm; Grid=0.1 cm    ATLAS DCS report:  23.02.1999

Gutter=0, Header,Footer=1.25 cm, CourNew-B10    Last revision   :  29.07.2001

## SCT-DCS GENERAL DATA TAKING ORGANISATION

S.G.Basiladze (MSU), R.Brenner (Uppsala)

- Introduction

===============

1.1. Software design aims

-------------------------

To make “scalable” software that allows to create a simple local test stations (1-2 Fieldbus boxes and several tens of channels) and from other hand to build the complex geographical distributed slow control systems that contain 5-6 hierarchical levels and thousands of the channels (Fieldbuses are on the second level of that systems, on the down level there are sensors and actuators).

The treating (access) organization should be oriented on a User, it means that

- the identificators of all the Detector parts (including sensors and  actuators) should be represented in the text form (as a Name);

- the hierarchical Names (the serial chain of local names from the every  level) should be used mainly;

- the Data should be represented in the ‘physical’ form (temperatures,  voltages, currents, etc.) and conversion from ‘raw’ ADC counts to  calibrated data should be performed on the “far” side (where Data are  taking) but not on the “near” side where User is requesting them.

All the manipulations with the nodes (and their branches) of hierarchical system structure (tree) should be based on a simple set of Functions such as Open/Close, Put/Get Data/Status, etc. This set of functions should be hardware and link protocol(s) independent and may/should be used on the any hierarchical level.

Created software should be platform independent – any computer under Unix, Linux, Windows (and VAX VMS) may be used in the system.

Flexibility

- all important for User and changeable parameters of monitoring and control  should be placed into the configuration file(s) and may be changed without  software recompiling;

- any type of Fieldbuses and Fieldbus nodes may be used in the system as well  as some specific interconnection protocols (PC parallel and serial ports,  VME bus, etc.) in addition to the main CAN-Open protocol.

Testing facilities

- the ability of a node to perform the requested action should be checked  before the command execution;

- every step of the every action should be checked (i.e. every Function,  every subFunction, ... lowFunction should return an error code);

- User should have ‘near’ side feedback from any request execution on the  ‘far’ side;

- two simulation mechanisms should be built into software

- Data simulation, the random values around Normal physical value should be

generated in this mode,

- Error simulation, the random errors  with desirable probability should be

generated  for the Functions (it allows  to test the stability  of system

operation).

1.2. Hierarchical trees and their levels

----------------------------------------

The main SCT DCS terms and definitions are described in this section. The general structure of SCT and its Slow Control (SC) system is shown in Fig.1.

Figure 1

UPPER            ATLAS          LEVELS

...                              ...

o                    o /

/()\ Expert     User /()

/\                   /\

HARDWARE:                       SOFTWARE:

SC Syst

Detector

Local Control                   DCS Requests

Station

Ethernet               TCP/IP

Host

Section

Satellite Control               DCS Functions

Station

PCI Bus                TCP/IP

Sector

CAN Controller                  CAN Procedures

Node

CAN Bus

“CAN Open”

Unit

Module

CAN Box

CAN Operations

Mini Link

Demu

Mulx

Channels

Chan

Chan

Actuator

Sensor

System Numbers ->   0 . . . . N                     N . . . . 0

of the Channels                   “Name Convertor”

Detector Tree – the hierarchical structure of SCT-detector itself (see Fig.1 from the right). It may contain up to 6 levels, the main of them are the following:

- Detector  level, includes SCT-detector as a part of ATLAS;

- Section   level, maybe the barrel cylinders or the forward tracker disks;

- Sector    level, the staves (parts of cylinder) or sectors (parts of disk);

- Module    level, the microstrip detector modules of a stave or disk;

- Sensor/Actuator level, the module channels for monitoring or control.

# DCS Tree - the hierarchical structure of Detector Control System (DCS) for SCT-detector (see Fig.1 from the left). It may contain up to 6 levels, the main of them are the following:

- Syst(em)  level, based on Local Control Station (LCS) – IBM PC and is a part

of ATLAS DCS;

- Host      level, the subparts of SCT DCS based on Satellite Control Stations

(SCS) – IBM PC;

- Node      level, PC controller cards for communications;

- Port      level, the ports of CAN-controller card, or PC serial or parallel

ports, or a port of Ethernet card;

- Unit      level, the devices that are connected to CAN-bus, or to PC serial

or parallel links;

- Chan(nel) level, the parts of Unit that get a monitoring data from one Sensor

or put control data to one Actuator.

Agent – tree node on the any level that has brunches to the down Agents (if level > 1).

## WARNING: 1. The term ‘Agent’ is used here instead of ‘node’ (of tree) because

the term ‘Node’ is occupied for the 4-th level of DCS Tree.

2. The CAN-node (box) is a Unit in this document.

The both structures have a common root and (on the down level) the equal number of channels, but the number of intermediate levels and the number of Agents on the every level may be different.

1.3. Agents Names and Addresses

-------------------------------

As it is well known, every Agent Iden(tificator) may be represented in the following forms:

- Name   , readable text string;

- Address, “numerical name” – set of numbers.

The Names are using for the Detector Tree (because they are oriented on SCT User) and the Addresses are using for the DCS Tree (they may be used by DCS Expert).

The hierarchical Idens only are used for the Names/Addresses because they reflect of Tree structure and due to it they are more convenient. The delimiter between the Local Idens is “.” (for both - Names and Addresses).

1.3.1. Local and System Numbers

It is supposed that DCS Tree logically has “full” and fixed structure: every Agent on some level has an equal number of subAgents (not all of them may be occupied physically).

Local Number  – a serial number of Agent in the group that belongs to one upper Agent.

System Number – a serial total number of Agent on its hierarchical level.

The terms: Host, Node, Port, Unit, Chan(nel) are used in DCS Tree for its hierarchical Agents definition and for their Local Numbers as well:

Host - the serial number of Power PC (if many);

Node - the serial number of CAN Controller card in one Host;

Port - the serial number of CAN Controller Port (or CAN bus) in one Node;

Unit - the serial number of CAN-box on the CAN-bus (for one Port);

Chan - the serial number of CAN-box channel.

The DCS Tree may be described by the levels number of numerical values. Every value defines the maximum possible number of subAgents (or branches) on the appropriate level: HOSTmax, NODEmax, PORTmax, UNITmax, CHANmax. For DCS Tree the System number of any Agent may be easy calculated:

for a Node    Nnum = (Host * NODEmax) + Node;

for a Port    Pnum = (Host * NODEmax  * PORTmax) +

(Node     * PORTmax) + Port;

for a Unit    Unum = (Host * NODEmax  * PORTmax  * UNITmax) +

(Node     * PORTmax  * UNITmax) +

(Port     * UNITmax) + Unit.

The system numbers are non-visible usually neither for User nor for Expert, but they are widely used in the software for access to Node and Unit structures.

###### The System Address (Iden) is represented as a hierarchical set of the Local Numbers of (upper) Agents:

Node System Iden (Addr):  Host . Node . tNOB . tNOB . tNOB

Unit System Iden (Addr):  Host . Node . Port . Unit . tALL

Chan System Iden (Addr):  Host . Node . Port . Unit . Chan

Two formal “numbers” – tALL and tNOB may be useful in the System Idens for addressing  to ALL the Agents of some level or to NOBody on that level and below; in this case a System Address has the fixed length.

1.4. The kinds of DCS information

Data – a portion of information from/to Sensor/Actuator; the Data may be

- Raw     , means the Data will be sent to (were taken from) channel directly,

- Physical, Data in float representation that converted from the Raw data by

using calibration constants.

Stat(us) – every Sensor or Actuator channel and upper Agents have 32-bits Control Status Register (CSR).

Info – the common term for Data and/or Status of a channel.

1.5. Multiplexing and demultiplexing in CAN

Every CAN message  on a CAN bus usually has  a Data Fragment that contains

the set of bytes (say – 8). One or two bytes  create  1 Cell  that is carrying

Data  from/to  one DCS Chan. Every CAN box  may have  several CAN frames  with

the individual  internal addresses (for instance, PDO1 and PDO2  in CAN Open);

it means  that DCS Chan-s  are organised inside CAN box  as internal two level

structure – Data Cells are combined in the CAN Fragments (frames).

The local serial number of DCS Chan in the CAN box  may be calculated in a

following way:

Chan = (Frag * CELLmax) + Cell,                    (1)

where CELLmax – maximum possible number of Cells in one Frag(ment).

Because the internal CAN box organisation may be very different (for instance, the ATLAS LMB box has one CAN cell  but it is sending Data  from 32 DCS Chan-s in  serial) the  internal structure  of  Multiplexors and Demultiplexors (i.e. frames and cells)  is “transparent”  for User (as it is shown in Fig.1) and for him one DCS Unit contains the DCS Chan-s only. The only place where frames and cells may be “visible” is the list of Chan-s in the Configuration file.

While sending the data into one DCS Chan only  some care should be taken  about other Data Cells  in the same CAN frame (they should be filled  by appropriate  Data  for other DCS Chan-s). The  way  was chosen  for obtaining  the  “frame coherence”  is  to keep  the copy of  CAN cells  in the software structures that are associated with the Unit.

2. The User means for Sensors parameters setting

================================================

Every Agent of SC system  needs usually  some  Configure information  from

the upper system level  before Running. The channels (down Agents) should have

the Calibration information also  for sending  the unified (physical) Data  to

the upper layers. The upper layer Agent should be able to get (set) the Status

information  about the State(s) of monitoring devices. The Synchro information

may be necessary also for co-operative work of all the DSC parts.

In principle, the inter-Agents information (Info) exchange may be driven

1) by Call  (on Info Request from the upper level);

2) by Event (on Info Sending from the down  level);

3) by Loop  (periodically).

In  the  Slow Control system  the first  or the third  mechanisms  may be used

mainly (the Event driven exchange needs Agent Guarding procedure in addition),

except some extra cases (from Interlock) where the second type have to be used

for obtaining the minimum detection time of a fault.

Every Request  should have a Response  in the TimeOut limits.  The TimeOut

value is a part of Config Info.

2.1. The Agents Status Files

----------------------------

In according with the DCS structure the Status Info is subdivided into the

several functional levels:

- the System Config Info  shows the DCS structure constants and what History

and Error files will be used in the Run;

- the Host Config Info defines  Input/Output  queues  lengths, TimeOut Values,

the Period of Monitoring, the number of Scanning Loops in the Run, etc.;

- the Node Config Info defines the physical (manufacture and production) names

the logical Idens and initial Status of DCS Nodes and their Ports;

- the Unit Config Info defines the same parameters for Units and their Chan’s;

the last 3 Config’s contain also  the System addresses  of every Agent  in DCS

structure.

Every Status Info (see Suppl.8.1) is represented  originally  as Microsoft Access Table  that may be changed by Expert  before a Run  and after it  every changed Table  should be exported  to the Bin-directory  as a plain text file. The Hosts must be placed in the Tables in the serial incremental order because every Table is reading only until the end  of found Host section. The end of all the tables is marked by the “99” (unreal) number of the Host.

2.2. The Naming and Calibration Files

-------------------------------------

The naming and calibration Info is placed in a Configuration File that is the base file for Expert. The Naming Table (see Supplement 8.2 from the left) includes  Names of  Detector sensors and actuators and CAN Addresses of DCS Chan-s that are associated with them.

2.2.1. The Detector Names

### Sensor Names

General:               Short:         Detailed:                 Short:

Alignment              Algm

Cooling                Cool

Current                Curr

High (Volt)  Curr         HivoCurr

Humidity               Humi

Ambient      Humi         AmbiHumi

Interlock              Lock

Hardware     Lock         HardLock

Software     Lock         SoftLock

Mechanics              Mech

OverCurrent            Ovcr

High (Volt)  Ovcr         HivoOvcr

Low  (Volt)  Ovcr         LovoOvcr

Pressure               Pres

Barometric   Pres         BaroPres

Temperature            Temp

Ambient      Temp         AmbiTemp

Cable        Temp         CablTemp

Cooling      Temp         CoolTemp

High Voltage Temp         HivoTemp

Low Voltage  Temp         LovoTemp

Module       Temp         ModlTemp

Shield       Temp         ShldTemp

Stave        Temp         StavTemp

Voltage                Volt

High         Volt         HighVolt

Low          Volt         Low_Volt

#### Actuator Names

(will be added)

#### Detector Names

Comments:

##### Module level  :                       Module

Sector level  :                       Stave                      (Barrel )

Sector                     (Forward)

Section level :                       Layer                      (Barrel )

Disk                       (Forward)

Detector level:                       SCT_Barrel

SCT_Forward

For instance, the full name of detector module Sensor may be:

SCT_Barrel . Layer04 . Stave03 . Module02 . HighVolt01

2.2.2. The Calibration Info

The Limit Table (see Suppl.8.3) includes information  about the Types of 6 possible Limits and appropriate Normal physical value. The 16 Limit Types  may be currently used.

The Calibration Table in a Config File (see Suppl.8.2 from the right) contains the Info  for conversion of Raw values to Physical ones. The number of calibration constants  now is equal to 4  and the current calibration formula for the first step is the following:

s = B*(x - A),                                                   (2)

where s   - is the sensor value (voltage, current, etc.),

x   - is the raw ADC count (integer),

A,B - are the first step calibration constants.

The following approximations may be used on the second step:

linear approximation

y = (s - C)/D,                       if s = D*y + C              (3.1)

square root approximation

y = [SquareRoot(C*C + 4D*s) - C]/2D, if s = D*y*y + C*y          (3.2)

logarithm approximations

y = C + [ln(s)/D],                   if s = exp[D*(y - C)]       (3.3)

y = 1 / {[1/C] + [ln(s)/D]},         if s = exp{D*[(1/y)-(1/C)]} (3.4)

where y   - is the physical value (float),

s   - is the sensor value, see (2),

C,D - are the second step calibration constants.

The “A” and “B” are compensating  the pedestal and the tolerance in conversion

factor of ADC; the “D” and “C” constants  may be used  for linear or nonlinear

interpolation of Sensor scale. If A=0, B=1, C=0, D=1 the raw values will be on the “output” (y = x) for calibration formula (3.1).

The complex sensors, for instance for humidity monitoring, may be described in Calibration Table also. The formula for calculating of the humidity is

h = H1/(H1 + H2)                                          (3.5,a)

Relative Humidity, RH is calibrated by:

RH = B*h - A.                                             (3.5,b)

The A,B-calibration constants  should be placed  in the record  for the second

sensor (H2) and in its  last column (Refer) should be presented  the reference

on the first sensor (H1) local number in the same Unit.

The Agents Status, Naming and Calibration Tables (or Files) are the internal specifications of SC system  and they  should be  non-changable (non-writable) for User.

2.4. The Selection Groups

-------------------------

The Data from monitored sensors may be selected by using the various criteria’s. Every criteria combines the proper channels into a

Group – the set of monitored Chan-s that are selected for Data extracting. For instance, the simplest way is to combine all the sensors of one type into one Group. The second way – to unite them basing on their “geographical area” community (for instance, for Ladder or Disk), etc.

The 8 selection Groups are possible in the current software. The 8-bits array is using in the CSR of any channel as a (linear) Group code; it means that every Chan may belong up to the 8 Groups simultaneously. The information about the Chan-s Addresses and their serial order inside the Group is stored in ‘Csct_gr1-8’ files (see Suppl.8.4).

NOTE: 1. The start Group number is ‘1’;

2. The start Order number is ‘1’ (‘0’ means the “channel absence” - may

be used as a mask);

3. In the Request attributes the Group number corresponds formally to

Chan number and the Order number corresponds formally to Input-Info

(see 3.3).

The Chan-s that are listed in the Group-files are included automatically in the set of monitored channels.

2.5. The Status Byte convention

-------------------------------

As was said before the every DCS Agent (on the Node, Port, Unit and Chan levels) has an unified “Control-Status Register” (CSR):

CAN Specific CSR |       Group  Code     |  R     W     S     E

|         |       (bit array)     |     |     |     |

|         |  |  |  |  |  |  |  |  |80|40|20|10| 8| 4| 2| 1  h

The meaning of R(ead), W(rite), S(tatus), E(rror) fields is shown below:

HexCode:

E    Bit_0,1 - Severity of Error: 0 - Normal (No Error),             0h

2 - Warning,                       2

1 - Alarm,                         1

3 - Fatal;                         3

S    Bit_2,3 - Operation mode   : 0 - Module is OFF System,          0

1 - Module is IN  System,          4

2 - Module is READY,               8

3 - Module is ACTIVE (Busy);       C

R: W:

W    Bit_4,5 - Write Abilities  : 0 - No Ability,                  00 00

R    Bit_6,7 - Read  Abilities  : 1 - By Call (from UppLevel),     40 10

2 - By Event(from LowLevel),     80 20

3 - Periodically.                C0 30

###### The “general” status information (2 low bytes) is writing to the every Agent CSR  from the Config Files during the Initialization (see Supplement 8.1).

In addition to 2 general status bytes there are 2 CAN specific CSR bytes  that may be used  in the different ways. They may be  written and/or read by using the special DCS “status access” functions.

3. The hierarchical structure of DCS functions

==============================================

###### As can be easy seen  from Fig.1  the CAN equipment  is only the part (down

level part) of the entire SC system  because CAN protocols can't cover all the

needs of a complex SC system. For instance:

- “CAN Open” protocol is defined in the boundaries of one CAN bus only,

- the possible number of CAN cells for one COB-Id is very limited.

But, of course, the standard CAN protocols and "CAN Open" in particularly  may

be included on the down level in the set of SC system protocols.

It means  that one (or several) additional function(s) level(s)  should be

used in the system. The base functions are: “Write” and “Read”  Data or Status information.

The current hierarchical organization  of the main DCS functions  is shown

in the Fig.2.

Figure 2

Name             SET       GET                                  Near side

What to do       Info      Info      External Buffer:           Request level

Data                                  Channels Data

Request    Request     Channels State

Expert

Way                  Network or Direct Link   ‘Copy’

<==================================================>

DCS  Trans/Action       DCS message:             Far side

Respond level

Segment  Length

‘Read’       Physical Data

Chan(s)  State

DCS Function                         -----

------ |  s   Manufacture

|  t    and Protocol

‘Write’     Status / Raw Data    i     Independent

General Structure   n      Level

U

CAN(Open)                          ------

Procedure                         ------ |  s   CAN Protocol

|  t    Oriented

CAN Specific       i     Level

Status Structure    n

U

CAN Operation                                    Manufacture

‘Send’   ‘Take’            Driver Level

The manufacture drivers  and/or  "CAN  Open" profiles  are the base (first

level) elements  of  DCS  Functions  hierarchy.  The  several  simple  CAN-box

Operations are combined in one CAN Procedure for changing of CAN modules State

or setting/getting the Data.  The specific (vendor defined)  Status structures

are using on this level.

Every Node/Unit on the next level  is represented by its Node/Unit General

Structure which contains the main changeable Node/Unit attributes (parameters)

and the Monitored Data (for Unit).  These Structures  are initialized from the

Config_Files when an Init Function of the proper level is called.

The next  is so-called (trans)Action level  where the CAN (or other)  Data

and Status are placed  into the Message buffer. There are  2 types  of the DCS messages:

- short ones, where only  one channel  Data or Status is sending as an integer

(raw) value,

- long ones, where the physical Data and Status information from all the

channels is sending.

In the last case the monitored Data  are converted  in the calibrated form and the message information is “compactizied” (see Sect.5). All the Data and Status requests are the short messages. The respond DCS message  maybe short or long. The format of DCS message  is described below, the message buffer on the “far” side is storing the Info about one Scan only  and it is rewriting periodically in the asynchronous Mode.

The last - "Request" level  permits to make  setting and getting  DCS Info by using  the Names  of Detector parts. Every Request  may be  a broadcast  or broadcall command for CAN hardware because a Detector Name may correspond to a group of the CAN channels.

There are  two possibilities  in the interconnections  between Request and

Respond levels (Fig.2):

- real TCP/IP Client/Server connection from near to far side (remote Mode),

- direct “near” to “far” internal connection (Net simulation - local Mode);

in the last case a DCS message is using just an internal Buffer for one Scan.

3.1. The “far side” DCS Functions and they numbers

--------------------------------------------------

Below is a list of the main DCS Functions:

0x10: int  InitDCShost(  int Host);

0x14: int  InitDCSnode(  int Host,  int Node, int Port);

0x18: int  InitDCSunit(  int Host,  int Node, int Port, int Unit);

0x20: int WriteDCSstatus(int Host,  int Node, int Por#, int Uni#, int Cha#,

ui  Wsts);

0x24: int  SendDCSstatus(int Host,  int Node, int Port, int Unit, int Chan,

ui  Wsts);

0x28: int  TakeDCSstatus(int Host,  int Node, int Port, int Unit, int Chan,

ui *Rsts);

0x2C: int  ReadDCSstatus(int Host,  int Node, int Por#, int Uni#, int Cha#,

ui *Rsts);

0x30: int WriteDCSdata(  int Host,  int Node, int Port, int Unit, int Chan,

ui  Wdat);

0x34: int  SendDCSdata(  int Host,  int Node, int Port, int Unit, int Chan,

ui  Wdat);

0x38: int  TakeDCSdata(  int Host,  int Node, int Port, int Unit, int Chan,

ui *Rdat);

0x3C: int  ReadDCSdata(  int Host,  int Node, int Port, int Unit, int Chan,

ui *Rdat);

0x48: int  WaitDCSevent( int Host,  int Node, int Port, int Unit, int Chan,

ui *Edat, ui *Ests);

0x4C: int  TakeDCSevent( uc *Host,  uc *Node, uc *Port, uc *Unit, uc *Chan,

uc *More, ui *Cdat, ui *Csts);

0x50: int CloseDCSunit(  int Host,  int Node, int Port, int Unit);

0x54: int CloseDCSnode(  int Host,  int Node);

0x58: int CloseDCShost(  int Host);

uc -> unsigned char, ui -> unsigned int

# => may be absent, if Port,Unit,Chan-number == tNOB -> Access To_NOBody

The ‘Write’ and ‘Read’ functions  are writing and reading  Data or full Status

Word  to/from Unit general structures; the ‘Send’ and ‘Take’ functions provide

in addition  the Data and CAN specific CSR exchange  between Unit’s structures

and CAN hardware if an appropriate DCS Chan is writable or readable (see 2.5).

The Functions  are manufacture  and CAN protocols  independent  and may be

widely used for many types of Buses and Serial Lines. All the Functions return

a "zero" or an Error Code.

3.2. CAN Message Structure

--------------------------

The DCS Action (see Fig.2)  is an equivalent  of the DCS Function  but, as

was said before, it is organized as internal DCS message  with the fixed “byte

ordered” platform independent format.

The request  (command - from  the left)  and  response (execute - from the

right) Action messages have the usual fields:

COMmand  Message  (to CAN)        EXEcute  Message         ( from CAN )

B00   :  Message    Number        Message Number+1  (exe_msg_num: even)

B01-02:  Message    Length        Message    Length (in Bytes         )

B03   :  To.. - Chan  Iden        From - Chan  Iden (or  VME address  )

B04   :  To.. - Unit  Iden        From - Unit  Iden (    :            )

B05   :  To.. - Port  Iden        From - Port  Iden (    :            )

B06   :  To.. - Node  Iden        From - Node  Iden (    :            )

B07   :  To.. - Host  Iden        From - Host  Iden

B08   :  What - Func  Iden        What - Func  Iden (CAN/VME Function )

B09-13:  From - Five Bytes        To.. - Five Bytes

B14-17:  Info - Reserve= 0        Info - Error Code (LowByte is in B14)

B18-21:  Info - Reserve= 0        Info - Error Addr (LowByte is in B18)

B22-25:  Date - Day, H,M,S        Date - Day, H,M,S (Day     is in B22)

B26...:  Data - of 1 Frame        Data - of 1 Frame (and Status info  )

Every Message starts  from a current (incremented)  non zero Number, it is

odd for COM_message  and even for EXE_message. The two next bytes  contain the

message Length identifier. Five Bytes  of an address (To..) are carried in the

Bytes_03-07. Function Code is placed  in "What to do" Byte (B08) of a Message.

Client is starting  by issuing of a Request Message (with the next odd number)

for Data/Status reading or writing (the request attributes  are placed  in the

Bytes_26-29).

Server can control of CAN modules and can execute commands from the Client

Requests. Server acknowledges every Message receiving (by incrementing B00 and

repeating of B03-13 in the Reply Message) and if there was  short Info request

- includes Read Data and/or Status starting from Byte_B26. In the case of some

error detecting  the Server puts  the first byte of an error code  in Byte_B14

and the second byte - in Byte_B15, in the next 2 bytes  the Error Address code

will be written.

Every Message  has 4 bytes  for Day(month)/Hour/Minute/Second Time stamp –

in the Bytes_B22-25.

3.3. The “near side” DCS Requests and Responds

----------------------------------------------

###### There are 2 near side high level subroutines for manipulating with the single channel Data and/or Status info (short messages):

int  Request(int Host, int  Node, int Port, int Unit, int Chan,

int Func,               /* The DCS FUNCtion                  */

unsigned  int  Info);   /* The Data/Status Info for an INPut */

int  CopyRespond(int Host, int  Node, int Port, int Unit, int Chan,

unsigned  int *Info);   /* Pointer for Data or Status OUTput */

The first string in the attributes describes DCS Address of the channel. All the general Func(tion)s that are listed in p.3.1 may be used in the Request(). The CopyRespond() subroutine just takes the reading Info from the EXE_message and may be used for checking the quality of Request execution.

Two additional SC-system functions may be requested also (see details in p.6):

0x80: int SetDCSstate(int Scod); - Set DCS System State

0x84: ui  GetDCSstate(void);     - Get DCS Global state parameters

There are special Requests for Info in calibrated and normalized forms:

0x90:     GetDCSinfo             - Function Code for Request

0x94:     GetDCSnormal           - Function Code for Request (see p.5.2)

The full Data & Stat information from all the channels of the last Scan is sending in the compact format on this request. After it the small (Group) portions of Info may be copied from EXE-message locally.

3.4. The types and functionality of SCT-DCS Server and Clients

--------------------------------------------------------------

There are 2 types of the Clients (remote Request Agents) in SCT-DCS:

- "Main Client"  that may be run  by Expert  only. It is able to generate any

request to the Server;

- "Look Client"  may be used by everybody. It is able  to send  "GetDCSinfo"

request only.

The Server  is starting by  the Local  Client as a “Child”, they both are

sitting in a Host computer. The “Parent” is a Main client for the server. The

Server executable program  should be prepared  before its call  and should be

put into the file with “Cserver.exe” name.

The other  clients (their number  may be  up to 30)  are subdivided  in 3

groups: they can get SC data  “on Call” (by sending request  every time  they

need)  or “on Event” (that is discovered in the system)  or Periodically (“on

Time” – regular). The “event” and regular clients should register  themselves

before data getting by sending to Server  a Request with  GetDCSinfo function

and with the Info value

- 0x88  for adding to Event list (0x84 – for deleting), see CSR bits  in 2.5,

- 0xC8  for adding to Periodical list (0xC4 – for deleting).

After it they will receive the proper data from the Server.

- Watch-dog Timer                                             Figure 3

“isolation”

Local Client

( Parent )

Main                                                   Look

Remote Client                                           Remote Client

.                                                       .

. “on Call”                                  “on Event” .

.   Group                Server                Group    .

.                       ( Child )                       .

Look                                                    Look

Remote Client                                           Remote Client

“on Time”

Group

Look                          Look

Remote Client     . . . .     Remote Client

The TCP/IP links are using for remote clients. All the links  are realized in

unblocking  manner (there is  no  indefinite  waiting time  during “read” and

“write” transActions ( see watch-dog timers in Fig.3 ), therefore  nobody can

cause the system hang-up.

The Server Loops (time sharing) organization is shown in Fig.4. The average Detecting Time:

- for Scanned modules   - tens or hundreds of sec ( Tmon / 12 );

- for Eventable modules - several msec (if Server is out of Scan).

The advantages of "Periodical Sending" mode in CAN ( Tsend  (Tmon / 2) ):

- CAN bus loading is more regular;

- there is no waiting time for respond, therefore a Scan time is very short;

- there is no CAN-boxes TimeOut and hang-up problems.

The CAN Unit  is sending  Data to Server  only if the monitoring procedure

was done successfully, in other case it returns an error.

Figure 4

New Time

CAN

->                             Hardware

Time < Tmon ?                         CAN Request

CAN Respond

Is Client ?   Main

Make

Make Script    Quick               Server               one

Line        Loop                                     Scan

Loop

Is Event ?

Periodical

Sending

<-

###### CAN

Hardware

Send Scan data to

“Regular” Clients

3.4.1. Quick Loop procedures

In the 'Quick Loop' a Server is checking;

- the possible requests from Main and Look Clients;

- the 'Event List' that contains "look at me" Flags from the channels that are

able to send the CAN messages in some Extra Cases;

until the Period of Monitoring (Tmon) is not over.

3.4.1.1. Script Files

The manipulation with the Actuator channels needs sending some control Info in non-predictable order (at the time of Code compiling) in addition to getting Actuators Data and Status from addresses that may be varied. The Info exchange protocol may be based on some other functions than simple numerical Value Write/Read (see p.3.1). For instance, it may be based on Text String Sending/Taking (Chiller machine) and maybe very slow (needs several seconds waiting after every Request). The special system subroutine was created to meet these requirements, it is able to read and to execute the

Script - serial set of commands that are represented in the form of a text lines. The important new function in a Script is ‘WaitMsec’ (milliseconds). One Script Line is executing in one Quick Loop cycle, the next Line is reading on the next cycle only. If ‘WaitMsec’ is presented in the Line the incrementing of Sript Lines will be postponed till wait interval will be over (the Script itself will be transparent for Quick Loops).

The Func(tion)s that may be used in the Script Requests and their “text format” (in the left columns) is shown below:

InitHost: func = INITdcsHOST            InitNode: func = INITdcsNODE

InitUnit: func = INITdcsUNIT            SendStat: func = SENDdcsSTATUS

ReadStat: func = READdcsSTATUS          TakeStat: func = TAKEdcsSTATUS

SendData: func = SENDdcsDATA            ReadData: func = READdcsDATA

TakeData: func = TAKEdcsDATA            SendStri: func = SENDdcsSTRING

TakeStri: func = TAKEdcsSTRING          MoveStri: func = MOVEdcsSTRING

ClosUnit: func = CLOSEdcsUNIT           ClosNode: func = CLOSEdcsNODE

ClosHost: func = CLOSEdcsHOST           WaitMsec: No Request on it

As may be seen from previous list the additional functions were created for Strings dealing:

0x40: int  SendDCSstring( int Host, int Node, int Port, int Unit, int Chan,

char *Stri);

0x44: int  TakeDCSstring( int Host, int Node, int Port, int Unit, int Chan,

char *Stri);

0x9C:      MoveDCSstring  - FunctCode for Far side Request.

Every letter of a String[i] is copied while Sending/Taking into appropriate Chan[i] of selected Unit structure. Taking is copied a String into sGET global buffer also. Due to it any Stri(ng) was read may be moved to any Unit.

The format of the Script Line (Suppl.8.5) is the following:

Request   Host  Node  Port  Unit  Chan  Func  Info/Stri

or

WaitMsec  Time  None  None  None  None  None  None

If some of the parameters are not necessary for the request (for instance, Node, Port, Unit, Chan for ‘InitHost’) they should be described as ‘None’. Any String may be commented by inserting ‘#’ before ‘Request’ or ‘WaitMsec’.

4. Error Handling

=================

The current principle  of DCS functionality is "to execute until the first Error" on the any  (Action, Function, Procedure, Operation) level except Agents closing actions. Due to it the System Error register contains the unified fields  for  Action/Function, Procedure and Driver errors and for general Error status:

Error Bits 0,1 - Severity of Error : 0 - Normal (No Error is even always),

2 - Warning,

1 – Alarm  (Fault is odd always),

3 - Fatal;

If the system is working in the "periodical" or closing mode the Error counter  may be used also. The Error register ERRc (ERR-code) and Error counter ERRn (ERR-number) have 32-bits length and ERRc has following bits distribution:

Hardware Code | Function | Procedure | Operation Errors | ES

11 bits    |  6 bits  |  7 bits   |       6 bits     | 2| 1 h

Error Counter

b

where ES is Error Severity.

There are no  Error printing  (drawing)  facilities  in the  DCS Functions

themselves (because they are on a "Far side"). The special set  of subroutines

was developed for the Errors indication  in the Text or Graphics modes (on the

Client/User side). Some examples of the Error Messages are shown below:

Function Errors:  Procedure errors:            Operation Errors:

..........       ...........                  ..........

..........       ..........                   ..........

" InitDCSnode   :  Open_CanObj()      failure   Write_Queue_OverFlow    "

" CloseDCSnode  :  CrNotifAnalyz()    failure   Read_Queue_Overflow     "

" InitDCSunit   :  CanCrNotifBox()    failure   Overflow_in_CAN_chip    "

" WriteDCSstatus:  CanSdoServ()       failure   Unknown_Overflow        "

" ReadDCSstatus :  CanCrNotifGuard()  failure   Data_just_read_was_old  "

" WriteDCSdata  :  CanNmtServGlobal() failure   Not_supported           "

" ReadDCSdata   :  CanBoxWrite()      failure   CAN_communication_stuff "

" WaitDCSevent  :  Can't find Unit    by Name   CAN_communication_form  "

..........       ...........                  ..........

..........       ...........                  ..........

The special set of Data/State Read/Write simulation Functions was created;

it allows to exclude CAN hardware  from SC system  for other DCS parts testing

and software debugging. Any errors  can be generated  artificially in a random

way in this Mode, it permits to check the system sensitivity to errors and the

system tolerance to them.

5. Data Compactization

======================

The monitoring calibrated Data  are usually represented as float variables

(see above), it means  that several tens of bits  are necessary  for the every

count storing. It is much more than the amount of information that contains in

the monitored Data. In according with the classical Shannon formula the amount

of information in one count is not more  than 6-7 bits  (the Data distribution

peak is relatively narrow - see details in the ATLAS Note No 142). The several

ways are possible  for reducing the Data buffer length in a Message  or in the

archive.

5.1. The Info Coding in DCS Message for monitored Data

------------------------------------------------------

There is the hierarchical set  of embedded segments  for Host, Node, Port,

Unit and Chan portions of Info in the CAN Message. The number of segments on the every level is equal to HOSTmax, NODEmax, PORTmax, UNITmax, CHANmax correspondingly. The every fragment  is starting  from its length (2 Bytes), due to it the CAN Message may be easy decoded and encoded.

5.1.1. Chan(nels) Info Compact Coding

The Chan Info contains  Data and  Status fields. They both  are “half Byte”

encoded: code starts  from  a current value length (the first half of Byte) and

then the value itself (float or integer) is represented in binary-decimal code.

###### For instance, for “zero value” coding  the only one Byte  is needed  for 32-bit

###### Integer (code = 01h) and 2 bytes are used (code = 0B 03h) for the Float.

HalfByte (hB) codes for Numericals:                       Byte

0..9 – Digits (Decimal),                   First  hByte     Length in hB/B

10  - Minus Sign,

11  - Float Dot;                          Third  hByte     Second hByte

Chars coding, if the First hByte:

12  - Char String, size = Length,         .  .  .  .  .    .  .  .  .  .

13  - Char String, size = Length + 15,

14  - Char String, size = Length + 30,        (NULL)       Last   hByte

15  - Char String, size = Length + 45.

The Name of Detector structure part may be coded also  as a Byte Array. The length of string (up to 60 Bytes) is in the ‘Length’ hByte.

The schematic structure of DCS-message is shown below:

Data Section Length

Host 0 Length

Node 0 Length

Port 0 Length

Unit 0 Length

Chan 0 Data   compact code

Chan 0 Status compact code

Chan 1 Data   compact code

Chan 1 Status compact code

.  .  .  .  .  .  .  .  .

.  .  .  .  .  .  .  .  .

Unit 1

Unit 2..

Port 1

Port 2..

Node 1

Node 2..

Host 1

Host 2..

5.2. Data Normalization

-----------------------

As we need to know  the detailed behavior  of monitoring parameter  around

its Normal value only the "three slope" scale may be used as it is shown in Fig.5.

Figure 5

Normalised Value

255-                           Monitored Data

Distribution

224-

192-

128-

Minm = Norm - (3*(Norm-FatL))

64-                                          Maxm = Norm + (3*(FatH-Norm))

32-

Raw Value

|                 |          |      |      |          |                  |

Minm             FatL       AlrL   Norm   AlrH       FatH               Maxm

For so-called Data Normalization procedure the Limits are standardized:

Alarm_Low is always equal to  64,   Alarm_High is always equal to 192,

Fatal_Low is always equal to  32,   Fatal_High is always equal to 224,

Minimum   is always equal to   0,   Maximum    is always equal to 255,

and Normal value is always equal to 128 - see Fig.5. As  a result  the 8 bits (1 Byte) only is necessary for monitoring Data (internal) representation.  Such format is very suitable for graphics Data out and for the further Data Compression.

If FatL=n*32,  AlrL=n*64,  Norm=n*128,  AlrH=n*192, FatH=n*224 ("n" is any

integer) are installed for the X-axe  then the conversion scale will be linear

(see LimiType "14" (n=1) and "15" (n=10) in the Calibration Table).

6. The System Subroutines

=========================

For using “by call” in higher level system software  the described set of

the programs may be represented as a single “functional box” with some inputs

and outputs, i.e. as the complex subroutine  that includes the set of Request and Respond near side commands (see 3.3).

Two  system operations  may be used  for sending  of the local  or remote

Requests to CAN hardware and receiving the Responds with Data/Status Info:

int SetGetByAddress(int *Host, int *Node, int *Port, int *Unit, int *Chan,

int *Func,     /* The DCS FUNCtion                  */

int *Iinp,     /* The Data/Status Info for an INPut */

float *Dout,     /* Pointer for Data   OUTput         */

int *Sout,     /* Pointer for Status OUTput         */

int *Eout)     /* Pointer for Error  OUTput         */

int SetGetByName(  char *Name, int *Func, int *Iinp,

float *Dout, int *Sout, int *Eout,

int *Numb)     /* The NUMBer of Chans found by Name */

The first subroutine is oriented on Expert Requests  because it deals with DCS

addresses  and  the second one – on  User Requests (Detector Name  is  in  the

arguments). The first subroutine returns (on Dout and Sout addresses) one Data

and one  Status value; the second one – the group  of Data  and Status  values

that was found  “under” Detector Name (it may be  Device, subComponent  or all

the Detector).

###### The writing and reading  are the examples  of “single” operation. For easy

control of the system resources more complex procedures are necessary that are

able to produce  the serial sequence of operations, such as: scanning, opening

and closing.

There are 2 System Procedures  that may be called via “SetGet” subroutines

for setting and checking  the total DCS Status  and for making  the monitoring

Scan:

0x80: int   SetDCSstate(int Scod);

where Scod - System Status CODe: 0x00 - Set OFF   System State,

0x04 - Set IDLE  System State,

0x08 - Set READY System State,

0x4C - Set ACTIVE State and Scan Data,

0xCC - Scan Data Periodically;

0x84: int   GetDCSstate(void); - returns of Scod.

Two additional System requests are defined for more specialized operations

by using the Detector's Names:

0x88: SetDCSdata  - for Data writing to the group of CAN channels

(makes WriteDCSdata & SendCANdata);

0x90: GetDCSinfo - for copying the results to Data and Status Buffers.

###### By using System Operations  and System Procedures any Extern Connection to the

SC System (via interprocesses mechanism or directly) may be done very easy.

6.1. The Testing Software

-------------------------

6.1.1. The Scanning Program as a LabView CIN

###### All the described functions  and subroutines  are written in ‘C’  but they

may be successfully used in the LabView environment as LabView CINs.

The DCS Scanning Program  is based on the SetGetByAddress() subroutine and

SetDCSstate(Scod) system function is called  on the every Scan. The functional

Module (see Fig.5) has 6 inputs: 5 - for Host, Node, Port, Unit, Chan numbers;

Iinf (input info) - for new Request and 2 outputs: Sout - an indicator  of the

Current  system  state  or  error,  Dout - for calibrated  or normalysed  data

getting.

###### The system or value current State (or Error info) is presented on the 1-st

output  in according with  unified Status (p.2.5)  and Error codes  convention (i.e. the Data are comparing with the Limits).

Data for Host(Node(Port(Unit(Chan))) are always available  from the second

output.

Figure 5

Status+Limits

Config.File    Calibr.File

Error

LabView

New                                            State

State              Scan Buffer                           User

Scanner                     Normalyser  Data

Channels Data                          Inp/Outp

Chan              Channels State

Address

o /

/()

/\

newState+ChanNumber           happy User

If Func=SETdcsSTATE and Iinf=4Ch the program is scanning  one time all the Sensors (of all the Units, of all the Ports, of all the Nodes, of all the Hosts) that are included in the Configure Table, and puts Data and Status values into the DCS-message.

If Func=GETdcsINFO the full Data & Stat message is sending to “near side”.

If Func=COPYgrpINFO the program is making a copy of Chan info that is numbered in ‘Iinf’ (the Group is pointed in ‘Chan’ section of Address).

If Func=COPYchaINFO the addressed Chan Info is copying.

6.1.2. The Testing programs

This scanning routine  is also a part of 2 Testing programs, the first one represents the results in the Text  and the second one - in the Graphics form. By using  two-level menu  in these programs  any channel  may be  selected for Data/Status writing from the keyboard. The scanning results may be watching on the screen  for the predefined group of the channels. The ‘Csct_gr1.txt’  file (see Supplement 8.4) is using for predefinition (selecting) of the Chan-s that are desirable demonstration (watching).

7. Software that is available from MSU

======================================

Assuming that all DCS subsystems  will be based mainly  on CAN equipment,

the following set of CAN hardware modules was tested:

- National Instruments CAN Controller card for IBM_PC;

- CAN-A and CAN-D slave boxes from "or" Industrial Computers;

- Local Monitor Box that was designed in ATLAS DCS group;

- Embedded LMB (ELMB) that was designed in ATLAS DCS group.

7.1. The list of Programs

-------------------------

The list of created testing Libraries and Programs is presented below.

Cversion.c - This file contains  all the Global "C" variables  and structures

and controls all the possible Oper_Modes.

It may be Compiled and Run  as the Server  or as the Client,

or as a Library (CIN for the LabView), or as the Test program in

the Text or Graphical Modes. The proper definitions must be done

in Define Modes Area.

If CANVHEAD and LIBRaries are defined  this File can be used

as the Header File.

Ctst_txt.c - CAN modules TeST in the TeXT mode, main program (for expert).

Tests all the CAN programs (Open/Close, Write/Read/Event) on the

Hardware  Independent  Trans/Action Level  in the Text  Terminal

mode.

Ctst_gra.c - CAN modules TeST in the GRAph mode, main program (for expert).

Tests all the CAN programs (Open/Close, Write/Read/Event) on the

Hardware Independent  Trans/Action Level  in the Graph  Terminal

mode.

Ccom_lib.c – This library contains the subroutines for execution of Script

described procedures.

Creq_lib.c - Provides CAN high level REQuests  for the System Initialisation,

Setting the States and Getting Slow Control Data.

Cact_lib.c - CAN ACTions LIBrary.

Provides Network (TCP/IP)  or Direct  Link  from  Client CAN/VME

program  to Server CAN/VME Functions  that deal with hardware or

with CAN/VME driver simulators.

May be used as

a} SIMulation library of network Actions (SIMa);

b) NETwork Client library (NETC),

c) NETwork Server program (NETS - in this case the name

of this File should be changed to Cact_ser.c).

Csys_lib.c - CAN SYStem LIBrary.

Makes the distribution of the calls  to the vendor oriented  CAN

subroutines  for the  System  Host Initialisation,  Setting  the

States and Getting the Data from Detector Control System Level.

Cnam_lib.c - Makes on call: Naming file reading  and then  Detector names and

CAN addresses structures structures filling; getting CAN Address

by Detector Name and getting Name by Address.

Cerr_lib.c - CAN ERRror LIBrary.

Provides all the necessary  Error Handling System subroutines on

the   Procedure   (hardware   dependent),   Function   (hardware

independent) and Action (remote control) Levels.

Cfun_sim.c - CAN FUNctions SIMulation library.

Provides  all  the necessary   ( Open/Close,  Write/Read/Event )

simulation  subroutines  for CAN  controllers  and boxes  on the

Function  (hardware  independent)  and Action  (remote  control)

Levels.

Comp_lib.c - COMPuter COMPatible LIBrary.

Creates    Platform   independent   Input-Output   Text-Graphics

facilities in "C".

NIca_lib.c - National Instruments CAN controller LIBrary.

Provides  all  the  necessary  ( Open/Close,  Write/Read/Event )

subroutines  that are based on  NIcan Controller drivers for the

Procedures (hardware dependent level).

NImb_lib.c - National Instruments & local Monitor Box LIBrary.

Provides  all  the  necessary   (Open/Close,   Write/Read/Event)

subroutines  for  Nican  drivers   and  for  LMB  boxes  on  the

Procedure (hardware dependent), Function (hardware  independent)

and Action (remote control) Levels.

NIel_lib.c - National Instruments & Embedded Local monitor box  LIBrary  with

the same possibilities as Nimb_lib.c.

NIor_lib.c - National Instruments & ORic box LIBrary.

Provides  all  the  necessary   (Open/Close,   Write/Read/Event)

subroutines  for  NTcan  drivers   and  for  CST  boxes  on  the

Procedure (hardware dependent), Function (hardware  independent)

and Action (remote control) Levels.

PCsp_lib.c – The library for IBM_PC Serial   Ports.

PCpp_lib.c – The library for IBM_PC Parallel Ports.

All the DCS settings and calibration constants are reading from the proper

Files during the Host Initialization. These files are generated from Microsoft

ACCESS DataBase Tables:

Csct_sys   - CAN (ATLAS) SCT SYStem       parameters,

Csct_hst   – CAN (ATLAS) SCT HoSTs        parameters,

Csct_nod   – CAN (ATLAS) SCT NODes        parameters,

Csct_uni   – CAN (ATLAS) SCT UNIts        parameters,

Csct_lim   – CAN (ATLAS) SCT LIMits types parameters,

Csct_cnf   - CAN (ATLAS) SCT naming and calibration CoNFig info.

Csct_gr1-8 – CAN (ATLAS) SCT Scan selection groups.

Csct_ini   - Script that is executed on the end of initialization.

Csct_run   - Script that is executed in the Quick Loop.

Csct_off   - Script that is executed on the start of system closing.

The both (text and graphics) test programs may be used for any combination

of  the  working  and  simulation  libraries.  After  choosing  the  necessary

combination  of libraries (in the Cversion.c File) the proper  version code is

generating (the first compiler operation). Version code "register" contains  3

fields - Hardware (8 bits),  Simulation (4 bits)  and  Terminal mode  (4 bits)

linear Codes:

| SystCode         HardCode         |   SimuCode   |Term|       ShowCode    |

|         |see Hardware Version Code|  A    F    D |  M |   A    S    D    E|

|8000|4000|2000|1000|800|400|200|100|  80|  40|  20|  10|   8|   4|   2|   1|h

|                                   |

Simulation CODe:          Terminal Code:                  Show CODe:

D - Driver    Simu/Work,  If M=1 -> Text  terminal mode,  E - Errors Show,

F - Function  Simu/Work,  If M=0 -> Graph terminal mode;  D - Data   Show,

A - Action    Simu/Work;  System Code:  00 - ATLAS,       S - Status Show,

10 - ZEUS;        A - Action Show.

The  place and role  of the every  program  may  be illustrated  with the

following picture:

Figure 6

Request Library                  Naming

or Test (Text/Graph Inp/Outp)       Configuration

Programs                      File

Net                                           Net

|- - -             CAN Actions Library         - - -|

System Commands Library

------

------ |

-------------     ------------  ||  ---------------

NIcan            CAN       |

Controller         Boxes           Agents Status

Library        Libraries              and

Calibration

Hardware  Simulation               Files

Library

NT-CAN          orIC-CAN             ELMB-CAN

Controller         Boxes      ....      Boxes

CAN   |                |                   |  CAN

The Status and Data ShowTerminal Modes are using for debugging of software

and hardware. All the software is written  in "platform independent" manner (it

may be run on  IBM-PC, Unix and VAX  machines) - the special set  of Text/Graph

Input/Output libraries was developed for obtaining this aim.

8. SUPPLEMENTs

==============

8.1. The Examples of Agents Status Tables

-----------------------------------------

8.2. The part of Configuration Table

------------------------------------

8.3. The Example of Limits Table

--------------------------------

8.4. The example of Group selection Table

-----------------------------------------

8.5 The example of Script (command) File

----------------------------------------