Development Facility

Manufacturing Procedures

###### ATLAS

###### Cooling Block High

1mm channel 6.5mm deep.

##### Procedure (for Steve Temple 21.12.00 )

- Machine tool XYZ 1500PRO CNC mill.

- Program generated by AGE software.

- Cutter speed 2000 rpm.

- Cutter type, 1mm long series carbide.

- Feed rate xy  82mm/min.

- Feed rate z 100µ increments.

- Cycle time 25mins approx.

##### Comments

- Carbon material cut well, with no sign of tool deflection at these feed rates.

- Small amount of tool wear after machining two components.

- With further optimisation of programme cycle time and tool wear can be improved.

- Using fixtures to hold a number of blocks, will make this a relatively straight forward machining task.

- Carbon material can cause problems with machines electronics.( use vacuum cleaner )