# ATLAS SCT Module Test Report

MODULE NAME: FR_K3-152

DATABASE S/N:

# Components

|  | Database S/N | Manufacturer S/N | Encapsulant | Comment |
| --- | --- | --- | --- | --- |
| Baseboard |  |  | 5µm copper |  |

| Detectors | Database S/N | Manufacturer S/N | Ileakage (nA) | Cinterstrip (pF) | Pinholes | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Top far |  | CiS3392 06 |  |  |  |  |
| Top near |  | CiS3386 03 |  |  |  |  |
| Total | - | - |  | 16 |  | - |
| Bottom far |  | CiS3392 08 |  |  |  |  |
| Bottom near |  | CiS3385 18 |  |  |  |  |
| Total | - | - | 8000 (total) | 18 |  | - |

|  | Database S/N | Manufacturer S/N | Comment |
| --- | --- | --- | --- |
| Hybrid | KIII-152 | Dyconex | Windows underneath chips |

| Chips | Database S/N | Batch | Wafer | X | Y | xeff | nDead | Non Trim | Gain | Offset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M0 |  | 29476 | ID14 | 1 | 3 | 1 | 1 | 2 | 54.4 | 3.8 |
| S1 |  | 29476 | ID14 | 1 | 8 | 2 | 2 | 1 | 54.1 | 3.3 |
| S2 |  | 29476 | ID14 | 1 | 9 | 1 | 1 | 1 | 51.9 | 3.1 |
| S3 |  | 29476 | ID14 | 1 | 13 | 2 | 2 | 3 | 56.2 | 1.0 |
| S4 |  | 29476 | ID14 | 1 | 15 | 0 | 0 | 2 | 56.1 | -2.4 |
| E5 |  | 29476 | ID14 | 2 | 5 | 0 | 0 | 5 | 52.2 | 4.1 |
| M8 |  | 29476 | ID14 | 2 | 7 | 1 | 2 | 4 | 55.0 | 4.6 |
| S9 |  | 29476 | ID14 | 2 | 11 | 1 | 1 | 2 | 52.6 | -4.4 |
| S10 |  | 29476 | ID14 | 2 | 12 | 2 | 1 | 5 | 55.6 | -2.5 |
| S11 |  | 29476 | ID14 | 2 | 14 | 0 | 0 | 2 | 54.4 | -1.9 |
| S12 |  | 29476 | ID14 | 2 | 15 | 0 | 0 | 5 | 54.1 | 1.2 |
| E13 |  | 29476 | ID14 | 2 | 17 | 0 | 1 | 2 | 56.9 | 3.5 |
| Total | - | - | - | - | - | 10 | 11 | 34 | - | - |

| Capacitors | #locations | C | Vendor | Part No. | FResonance |
| --- | --- | --- | --- | --- | --- |
| HV decoupling | 4 | 10 | Murata | GHM1530B103K630 |  |
| Large LV | 4 * 2 | 330 |  |  |  |
| Small LV | 7 * 2 | 100 | Murata | GRM39Y5V104Z25 |  |
|  |  | nF |  |  | MHz |

| Miscellaneous | Vendor | Part No. | Comment |
| --- | --- | --- | --- |
| Temp. Sensors |  |  |  |

# Construction Details

| Adhesives | Vendor | Part No. | Comment |
| --- | --- | --- | --- |
| Chip Glue | EPO-TEK | EE129-4 | DGND-AGND connected |
| Det. Elec. Glue | EPO-TEK | EE129-4 |  |
| Det. Therm. Glue | Ciba | Araldite 2011 | Boron nitride filled |

| Bonding | #locations | # bonds / location | Comment |
| --- | --- | --- | --- |
| AG-DG connections | 12 | 1 | Solid conductive glue connection under each chip |
| Detector Backplane | 1 | 3 |  |
| Strip Bias: detector to fanin | 2 | 2 |  |
| Strip Bias: fanin to AG | 2 | 2 |  |

Were any components changed during production?  If so, give brief details here:

# Operating Conditions

|  | Default | Actual | Comment |
| --- | --- | --- | --- |
| Vcc | 3.5V | 3.57V | on support card |
| Vdd | 4.0V | 4.0V | on support card |
| Vdet | 100V | 100V |  |
| Icc | - | 825mA |  |
| Idd | - | 430mA |  |
| Idet | - | 7.7µA |  |
| Bias Current | 267A | 267A |  |
| Shaper Current | 30A | 30A |  |
| Compression Mode | 1 (LEVEL XIX) | 4 (Test XXX) |  |
| Edge Detect | 0 (OFF) | 0 (OFF) |  |
| Box Grounding | AG at PCB | AG at Pigtail |  |
| Box Cooling | Fan | Cooled N2 |  |
| Module Temp | < 45C |  |  |

# Trimming

|  | Default | Actual | Comment |
| --- | --- | --- | --- |
| Trim Method | Minimisation of n Non Trim | Minimisation of RMS | Alternate: Minimisation of RMS |
| Trim VCAL |  | 20mV |  |
| Target Value |  | 200mV |  |
| n Non Trim (link 0) | - |  |  |
| n Non Trim (link 1) | - |  |  |
| n Non Trim (total) | - | 45 |  |
| RMS of vt50 distribution after trimming | - | 198.6 |  |

# Results

|  | M0 | S1 | S2 | S3 | S4 | E5 | Link 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gain (2fC to 3fC) | 61.2 | 58.4 | 58.2 | 60.6 | 60.4 | 57.9 |  | mV/fC |  |
| Noise @2fC | 16.6 | 15.8 | 15.4 | 15.5 | 15.5 | 15.6 |  | mV |  |
| Noise @2fC |  |  |  |  |  |  |  | fC |  |
| Noise @2fC | 1695 | 1694 | 1653 | 1594 | 1603 | 1685 |  | ENC |  |
| Stability point | =offset | =offset | =offset | =offset | =offset | =offset |  | mV |  |
| Offset (by Noise Occupancy) | 130.1 | 124.8 | 130.9 | 118.6 | 127.1 | 128.5 |  | mV |  |
| RMS of dist. of NO offsets | 12.76 | 14.7 | 13.56 | 12.48 | 12.40 | 11.73 |  | mV |  |
| Stability - Offset |  |  |  |  |  |  |  | mV |  |
| Stability - Offset |  |  |  |  |  |  |  | fC |  |
| n Non Trim |  |  |  |  |  |  |  |  |  |
| n Masked | 24 | 3 | 3 | 4 | 1 | 5 |  | - |  |
|  | M8 | S9 | S10 | S11 | S12 | E13 | Link 1 | Overall |  |
| Gain (2fC to 3fC) | 59.0 | 57.2 | 59.7 | 58.6 | 57.8 | 59.5 |  |  | mV/fC |
| Noise @2fC | 16.5 | 14.8 | 15.9 | 15.7 | 15.3 | 16.2 |  |  | mV |
| Noise @2fC |  |  |  |  |  |  |  |  | fC |
| Noise @2fC | 1752 | 1618 | 1660 | 1677 | 1657 | 1701 |  |  | ENC |
| Stability point | =offset | =offset | =offset | =offset | =offset | =offset |  |  | mV |
| Offset (by Noise Occupancy) | 131.3 | 129.2 | 134.3 | 130.3 | 130.8 | 128.6 |  |  | mV |
| RMS of dist. of NO offsets | 13.69 | 11.31 | 13.14 | 12.82 | 12.40 | 13.46 |  |  | mV |
| Stability - Offset |  |  |  |  |  |  |  |  | mV |
| Stability - Offset |  |  |  |  |  |  |  |  | fC |
| n Non Trim |  |  |  |  |  |  |  | 45 |  |
| n Masked | 8 | 12 | 5 | 1 | 6 | 2 |  | 48 | - |