|  | HANDLING OF NONCONFORMING EQUIPMENT |  |  |
| --- | --- | --- | --- |
| ATLAS Project Document No: | Institute Document No. | Created : 29.09.1999 | Page: 1 of 10 |
| ATC-OQ-QA-5420 |  | Modified : | Rev. No .: 1.0 |

| Quality Assurance HANDLING OF NONCONFORMING EQUIPMENT Abstract This document establishes a policy and provides guidelines for the handling of nonconforming equipment produced by contractors, collaborating institutes, and CERN Divisions or Groups. The objectives are to ensure that nonconforming equipment is segregated from conforming equipment, and is prevented from being installed in ATLAS; to eliminate or minimise the reoccurrence of problems leading to nonconformance by implementing appropriate corrective and preventive actions; to ensure that all nonconformities are properly documented and traceable. |  |  |
| --- | --- | --- |
| Prepared by: M Mottier EST/ISS Marcel.Mottier@cern.ch Adapted and revised by: H. Schmuecker hellmut.schmuecker@cern.ch | Checked by: G. Bachy | Approved by: TCSG EB |
| Distribution List |  |  |

| History of Changes |  |  |  |
| --- | --- | --- | --- |
| Rev. No. | Date | Pages | Description of changes |
| Draft Releases | October - 1999 |  | Draft Reviews by TC |

Table of Contents

1	PURPOSE	3

2	POLICY	3

3	SCOPE	3

4	RESPONSIBILITIES	3

5	DEFINITIONS	4

6	NONCONFORMANCE GUIDELINES	5

6.1	IDENTIFICATION OF NONCONFORMITY	5

6.2	REPORTING	5

6.3	REVIEW AND DISPOSITION	6

6.3.1	Critical and noncritical nonconformities	6

6.3.2	Use-as-is of the EQUIPMENT	6

6.3.3	Repair, rework or rejection	6

6.3.4	Closing the nonconformity	6

7	CORRECTIVE AND PREVENTIVE ACTION GUIDELINES	7

8	RELATED DOCUMENTATION	7

9	NONCONFORMANCE REPORT FORM	7

# PURPOSE

To establish a policy, and to provide guidelines for the handling of nonconforming equipment produced by contractors, institutes and CERN Divisions and Groups, to ensure that nonconforming equipment is segregated from conforming equipment and is prevented from being installed in ATLAS.

To provide guidelines for establishing appropriate corrective actions aimed at eliminating or minimising the recurrence of problems leading to nonconformities.

To ensure that all nonconformities are properly documented and traceable.

# POLICY

Procedures for the tracking of nonconformities detected during the production of ATLAS systems, assemblies, sub-assemblies and parts shall be established and documented.

Equipment that does not conform to the specified requirements shall be marked as nonconforming, and the occurrence shall be recorded in a nonconformity report.

Nonconforming equipment shall be segregated from conforming equipment and equipment awaiting inspection or test, to prevent further unintended use until the appropriate disposition is decided.

Nonconforming equipment shall be reviewed by authorised personnel to determine whether it can be accepted "as-is", or whether it should be repaired, reworked or rejected.

Repaired or reworked equipment shall be re-inspected and re-tested to verify conformance with specified requirements.

Nonconformance markings shall be removed by authorised personnel only, after the review has determined that the uncorrected equipment may be used "as-is", or after repair or rework is complete. The decision to use a nonconforming equipment shall be documented.

The causes of nonconformities shall be analysed and appropriate action taken to avoid recurrence of problems

# SCOPE

The policy and guidelines apply to all materials, parts and equipment to be installed in ATLAS and manufactured and/or assembled by:

- contractors,

- collaborating Institutes,

- CERN Divisions and Groups.

They apply to all stages of fabrication and assembly, from raw material procurement until final inspection and test.

For the construction of ATLAS a number of collaborations have been organised. The policy and guidelines described in this document are applicable whether the client organisation is CERN or a collaborating Institute.

# RESPONSIBILITIES

Project Engineers (PE) in charge of ATLAS systems, assemblies, sub-assemblies or parts shall:

- Ensure that procedures established by the supplier for the handling of nonconformities fulfil the requirements of this document and are followed.

- Make sure that all personnel involved in the production of equipment, at CERN, Institutes and Contractors, is aware of the nonconformities procedures.

- Review the disposition of noncorformities.

- Participate in the analysis of the causes of noncorformities and initiate appropriate corrective and preventive actions.

Authorised inspectors in charge of inspection and test shall be responsible for identifying, segregating and reporting all nonconformities.

Supplier's Contract Engineers are responsible for the documentation of all nonconformities and for notification of the client organisation.

The ATLAS System/Sub-System Project Leaders shall be responsible for:

- The approval or rejection of the use without repair or rework ("use as-is") of nonconforming equipment where the nonconformity is critical.

- The approval or rejection of proposed corrective action in cases where that action might affect the required performance of ATLAS, its ability to operate safely, or its construction.

# DEFINITIONS

| Authorised Inspector | A person authorised by the client to carry-out defined inspection and test activities in the framework of a contract or other procurement agreement. |
| --- | --- |
| Client | The organisation responsible for a contract. It can be CERN or an Institute collaborating to the ATLAS Project. |
| Conformity | Fulfilment of all specified requirements. |
| Contract engineer | The engineer designated by the supplier to be responsible for the contract and its follow-up, including all contacts with the client organisation throughout the duration of the contract. |
| Critical nonconformity | Nonfulfilment of a specified requirement that may impair the ability of the equipment to achieve the required performance or may impair its ability to perform safely. |
| Deviation | Written authorisation to depart from the originally specified requirements for a product prior to its production. |
| Disposition | Action to be taken to deal with an existing nonconforming equipment in order to resolve the nonconformity. |
| Equipment | Any material, part, sub-assembly, assembly, sub-system, or system designed or provided for use in ATLAS. |
| Nonconformity | Nonfulfilment of a specified requirement. |
| Product | The result of activities or processes. A product can be tangible (e.g. assemblies or processed materials) or intangible (e.g. knowledge or concepts), or a combination thereof. |
| Rejected | Nonconforming equipment not to be used |
| Repair | Action taken on a nonconforming equipment so that it will fulfil the intended usage requirements although it may not conform to the originally specified requirements. |
| Rework | Action taken on a nonconforming equipment so that it will fulfil the specified requirements. |
| Supplier | In the context of this document the term supplier is used to designate an organisation manufacturing and/or assembling materials, parts and equipment for ATLAS. Such organisation may be: |
|  | - A company |
|  | - A collaborating Institute |
|  | - A CERN Division or Group |
| Traveller dossier | The set of printed reports documenting the inspections and tests performed on a product. The traveller dossier follows the product through its production, inspection, delivery and installation. |
| Use-As-Is | Acceptable use of a nonconforming equipment. |
| Waiver | Written authorisation to use or release equipment which does not conform to all specified requirements. |

# NONCONFORMANCE GUIDELINES

The procedure for handling nonconforming equipment is shown in annex A.1.

## IDENTIFICATION OF NONCONFORMITY

If, in the course of the execution of an inspection or test procedure, a nonconformity is detected, the inspector shall clearly mark the nonconforming equipment by applying a nonconformity label on the equipment 's traveller dossier.

Where practical, nonconforming equipment shall be segregated from conforming equipment and equipment awaiting inspection or test by storing it in a clearly separated area.

## REPORTING

Once identified, the nonconformity shall be documented in a nonconformity report.

Important chatacteristics affecting the capability of the equipment to meet specified requirements shall be identified. The relationship of cause and effect should be determined, with all potential causes considered. The results of the investigation shall be recorded on the nonconformity report.

The nonconformity report shall contain, as a minimum, the following information:

- A nonconformity number.

- The date the nonconformity is open (identified).

- The number of the inspection or test procedure.

- The step in the inspection or test procedure where the problem occurred.

- The document number of the inspection or test report.

- The identification number of the equipment.

- A description of the nonconformity.

- The name and signature of the inspector.

When completed, the report shall be signed by the inspector and forwarded to the PE in charge of the contract. A copy shall be attached to the traveller dossier.

The nonconformity shall be reported to the Project Engineer in charge of the contract in the client organisation.

## REVIEW AND DISPOSITION

### Critical and noncritical nonconformities

All nonconformities that may have an impact on the equipment performance, durability, interchangeability, interface to other ATLAS systems, health or safety are categorised as critical nonconformities.

All nonconformities that are not evaluated to be critical as defined above are categorised as noncritical nonconformities.

### Use-as-is of the EQUIPMENT

The PE in charge of the contract, with the assistance of the PE in charge of the ATLAS system to which the equipment belongs, shall review nonconformities to determine what disposition should apply.

Possible dispositions include:

- Repair.

- Rework.

- Use-as-is.

- Reject.

The review shall first determine if the equipment may be used "as-is". If that is the case, the impact of the nonconformity shall be evaluated.

The "use-as-is" of equipment with a noncritical nonconformity shall be approved or rejected by the PE in charge of the contract. The decision shall be recorded on the nonconformity report.

The "use-as-is" of equipment with a critical nonconformity shall be submitted to the ATLAS System/Sub-System  Project Leader for review and approval or rejection. The decision shall be recorded on the nonconformity report.

When the decision taken is to authorise the "use-as-is" of the equipment, the completed nonconformity report is used as a waiver authorising the supplier to go ahead with the further processing of the equipment. The nonconformity is then closed.

### Repair, rework or rejection

When it is decided that the equipment cannot be "used-as-is", the PE in charge of the contract shall determine whether the equipment may be repaired/reworked or whether it must be rejected.

If it is decided that repair or rework is possible, the impact on cost, performance and schedule has to be evaluated.

The decision shall be recorded on the nonconformity report. If the decision is to reject the equipment, the nonconformity procedure is then closed. If the decision is to repair or rework, the nonconformity remains open until the inspection or test is repeated successfully.

Repaired or reworked equipment shall be re-inspected or re-tested to verify conformance with specified requirements.

Rejected equipment shall be removed from the production area.

### Closing the nonconformity

Nonconformity labels shall be removed by authorised personnel only, after it has been decided that the uncorrected equipment may be used "as-is", or after repair or rework is complete and the equipment has satisfied all specified requirements.

Appropriate measures shall be taken to prevent unintended use or installation of nonconforming equipment. This may require an assessment of:

- Previously manufactured lots of identical equipment that may be waiting delivery, or stored or already installed.

- Other equipment designed or manufactured in a similar way as the equipment found to be nonconforming.

# CORRECTIVE AND PREVENTIVE ACTION GUIDELINES

Action shall be taken, as appropriate, to determine the cause of critical, recurring, trend or pattern nonconformities and to develop and implement a remedy that will prevent the recurrence of such nonconformities. This may result in changes to equipment specification, or to production, inspection and test, handling, storage, installation procedures.

The decision to initiate any corrective action shall also be based on an evaluation of the adverse cost and schedule impact of the nonconformity relative to the cost and difficulty of its correction.

Action shall be initiated to a degree appropriate to the magnitude of the problem.

The PE in charge of the contract has the responsibility for initiating appropriate corrective or preventive actions.

Critical cases, such as those that might affect the required performance of ATLAS, its ability to operate safely, or its construction schedule shall be referred to the ATLAS Management for review and approval.

# RELATED DOCUMENTATION

[  ]	ATL-OQ-QA-5410		Manufacturing and Inspection of Equipment

# NONCONFORMANCE REPORT FORM

| Identification |  |
| --- | --- |
| Originator | Individual who identified the nonconformance |
| Date | Date the nonconformance was identified. |
| Report Number |  |
| Year/Code/Number | Enter the 2-digit year; use two letter department/division/project code as reference and a unique identifying number. |
| Incoming/In Process/Source/Other | Check the appropriate box if the nonconforming item/service was found during: - incoming inspection from the supplier, - during an in-process operation, - during a source inspection at a supplier, - other. |
| Contract Number | Enter if item was acquired through a Contract Division or an Intra-Laboratory agreement. |
| Part Name | Enter the part name as it appears in the title section of the drawing, specification, or supplier's product description document. |
| Part Identification Number | Ref. ATC-OQ-QA-2040 |
| Qty. | Enter the quantity of the item. |
| Nonconformance Description | Describe the nonconformance; ensure the applicable requirements, planned activities, procedures, specifications, drawing, standards, serial numbers, etc. are noted. Describe what was or could be affected by the nonconformance, and a justification for why the item doesn't meet the planned requirement. |
| Apparent Cause | Describe what caused the nonconformance, such as the events leading up to the nonconformance, the background, operator error, inadequate procedures or policies. The extent of the analyses should be commensurate with the importance or significance of the problem. |
| Steps to Prevent Inadvertent Use of the Item or Process | Describe how the item or process was identified and separated from other items. Indicate any actions taken to mitigate any environment, safety, and health impacts, if applicable. |
| Corrective/Preventive Action and Disposition |  |
| Planned Corrective Action on item | Describe what action will be taken with the item, including the completion dates and responsible staff for each action. |
| Planned Corrective/Preventive Action on Process or Design Independent verification required? Yes ___ No ___ | Describe what action will be taken with the process or design, including the completion dates and responsible staff for each action. Describe what actions are needed to prevent recurrence of the identified nonconformance, such as process improvement, procedure revisions, training plan, etc., and include completion dates and responsible staff for each action. Independent verification is performed by a Quality Representative or other staff independent of the performance of the corrective action. |
| Entered into Production Data Base ? Yes ___ No ___ | Note the decided action on whether to track nonconformance |
| Disposition | Note the decided action taken on the item/service. |
| Person(s) Responsible for the Corrective/Preventive Action and Disposition | Name of Project Engineer completing the corrective/preventive action and disposition. |
| Approval of Corrective/Preventive Action and Action Disposition | System/Sub-system Project Leader or designee/date. |
| Closing the Nonconformance |  |
| Planned Corrective Action has been Completed | Name/date (PE or designee). |
| Independent Verification has been Completed | Name/date (this cannot be the person doing the work). |
| Distribution | Record copy to Data Base inform Division of Contracts and Procurement. |

| Nonconformance Report Form |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Identification |  |  |  |  |  |  |
| Originator |  |  | Date | Report Number |
|  Incoming  In Process  Source  Other |  |  |  |  |  |  |
| Contract. No. | Part Name | Part Identification Number | Qty. |
| Nonconformance Description |  |  |  |  |  |  |
| Apparent Cause |  |  |  |  |  |  |
| Steps to Prevent Inadvertent Use of the Item or Process |  |  |  |  |  |  |
| Corrective/Preventive Action and Disposition |  |  |  |  |  |  |
| Planned corrective action on item |  |  |  |  |  |  |
| Planned Preventive Action on Process or Design |  |  |  |  |  |  |
| Independent verification required? Yes  No  |  |  |  |  |  |  |
| Entered into Production Data Base Yes  No  |  |  |  |  |  |  |
| DISPOSITION Use-as-is , Repair , Rework , Return to Vendor , Scrap, Other |  |  |  |  |  |  |
| PE Responsible for the Corrective/Preventive Action and Disposition _________________________________________ PE Name Date |  |  | Approval of Corrective/Preventive Action ______________________________________________ PL or Designee Date |  |  |
| Closing the Nonconformance |  |  |  |  |  |  |
| Planned Corrective/Preventive Action Has Been Completed _________________________________________ PE Name Date |  |  | Independent Verification Has Been Completed ______________________________________________ Name Date |  |  |
| Distribution: |  |  |  |  |  |  |

Annex A.1 Nonconformance procedure